Instructions to the Contractor:

- Please familiarize yourself with the SWPPP document and fill in any blanks with additional information such as Operator/Contact/ Manager Names, Phone Numbers and E-Mails, as well as Approximate Start & End Dates, all other Dates, Training Personnel, and/or any other needed information throughout the document.

- Please place a copy of the Material Safety Data Sheets in Appendix E.

- Please fill out the NOI included in appendix D and include a copy of the letter from the state notifying you of their receipt of your complete NOI and application in Appendix D.

- Fill in information in the appropriate location on page v of this document now and any time there are revisions to the document.

- Please place this page in appendix C after completing the above items.

Stormwater Pollution Prevention Plan

for:

The Summit @ Ski Lake Number 11

Via Cortina and Via Monoco

Huntsville, Utah  84317

Operator & SWPPP Contact:

Company:

Name:

Address:

City, State, Zip Code:

Telephone Number:

Fax/Email:

SWPPP Preparation Date:

December 3, 2010

Estimated Project Dates (TBD by Owner):

Project Start Date:  __ __ / __ __ / __ __ __ __

Project Completion Date: __ __ / __ __ / __ __ __ __

Contents

SECTION 1: SITE EVALUATION, ASSESSMENT, AND PLANNING	1

1.1	Project/Site Information	1

1.2	Contact Information/Responsible Parties	1

1.3	Nature and Sequence of Construction Activity	3

1.4	Soils, Slopes, Vegetation, and Current Drainage Patterns	3

1.5	Construction Site Estimates	3

1.6	Receiving Waters	3

1.7	Site Features and Sensitive Areas to be Protected	3

1.8	Potential Sources of Pollution	4

1.9	Endangered Species Certification	4

1.10	Historic Preservation	4

1.11	Applicable Federal, Tribal, State or Local Programs	4

1.12	Maps	4

SECTION 2: EROSION AND SEDIMENT CONTROL BMPS	5

2.1	Minimize Disturbed Area and Protect Natural Features and Soil	5

2.2	Phase Construction Activity	5

2.3	Control Stormwater Flowing onto and through the Project	6

2.4	Stabilize Soils	6

2.5	Protect Slopes	6

2.6	Protect Storm Drain Inlets	6

2.7	Establish Perimeter Controls and Sediment Barriers	6

2.8	Retain Sediment On-Site	7

2.9	Establish Stabilized Construction Exits	7

SECTION 3: GOOD HOUSEKEEPING BMPS	9

3.1	Material Handling and Waste Management	9

3.2	Establish Proper Building Material Staging Areas	9

3.3	Designate Washout Areas	10

3.4	Establish Proper Equipment/Vehicle Fueling and Maintenance Practices	10

3.5	Control Equipment/Vehicle Washing	10

3.6	Spill Prevention and Control Plan	10

3.7	Any Additional BMPs	12

3.8	Allowable Non-Stormwater Discharge Management	12

SECTION 4: SELECTING POST-CONSTRUCTION BMPs	13

SECTION 5: INSPECTIONS	14

5.1	Inspections	14

5.2	Delegation of Authority	14

5.3	Corrective Action Log	14

6.1	Recordkeeping	15

6.2	Log of Changes to the SWPPP	15

6.3	Training	15

SECTION 7: FINAL STABILIZATION	16

SECTION 8: CERTIFICATION AND NOTIFICATION	17

SWPPP APPENDICES	18

Appendix A – General Location Map

Appendix B – Site Maps

Appendix C – Construction General Permit

Appendix D – NOI and Acknowledgement Letter from EPA/State

Appendix E – Inspection Reports

Appendix F – Corrective Action Log

Appendix G – SWPPP Amendment Log

Appendix H – Subcontractor Certifications/Agreements

Appendix I – Grading and Stabilization Activities Log

Appendix J – Training Log

Appendix K – Delegation of Authority

Appendix L – Standards and Specifications for Selected BMP’s

Appendix M – Additional Non-Selected BMP’s

CIVIL ENGINEER CERTIFICATION:

I certify under penalty of law that this document and all attachments were prepared under my direction or supervision in accordance with a system designed to assure that qualified personnel properly gathered and evaluated the information submitted.  Based on my inquiry of the person or persons who manage the system, or those persons directly responsible for gathering the information, the information is, to the best of my knowledge and belief, true, accurate and complete.  I am aware that there are significant penalties for submitting false information, including the possibility of fine and imprisonment for knowing violations.

Mark E. Babbitt, P.E.

Project Manager

Revision Schedule

This storm water pollution prevention plan (SWPPP) should be revised and updated to address changes in site conditions, new or revised government regulations, and additional on-site storm water pollution controls.

All revisions to the SWPPP must be documented on the SWPPP Revision Documentation Form, which should include the information shown below.  The authorized facility representative who approves the SWPPP should be an individual at or near the top of the facility’s management organization, such as the president, vice president, construction manager, site supervisor, or environmental manager.  The signature of this representative attests that the SWPPP revision information is true and accurate.  Previous authors and facility representatives are not responsible for the revisions.

SWPPP Revision Documentation Form

| Number | Date | Author | Company Representative Signature |
| --- | --- | --- | --- |
| 0 | 12.3.2010 | Ryan Bingham |  |
| 1 |  |  |  |
| 2 |  |  |  |
| 3 |  |  |  |
| 4 |  |  |  |
| 5 |  |  |  |
| 6 |  |  |  |
| 7 |  |  |  |
| 8 |  |  |  |
| 9 |  |  |  |
| 10 |  |  |  |

# SECTION 1: SITE EVALUATION, ASSESSMENT, AND PLANNING

## 1.1	Project/Site Information

| Project/Site Name: The Summit @ Ski Lake Number 11 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Project Street/Location: Via Cortina and Via Monoco |  |  |  |  |  |
| City: Huntsville |  | State: Utah |  | ZIP Code: 84317 |  |
| County or Similar Subdivision: Weber County |  |  |  |  |  |
| Latitude/Longitude (Use one of three possible formats, and specify method) |  |  |  |  |  |
| Latitude: | Longitude: |  |  |  |  |
| 41º14'37'' N | -111º47'09'' W |  |  |  |  |
| Method for determining latitude/longitude: |  |  |  |  |  |
| USGS topographic map (specify scale: ) |  |  | EPA Web site |  | GPS |
| Other (please specify): Google Earth |  |  |  |  |  |
| Is the project located in Indian country? Yes No |  |  |  |  |  |
| If yes, name of Reservation, or if not part of a Reservation, indicate "not applicable." |  |  |  |  |  |
| Not applicable. |  |  |  |  |  |
| Is this project considered a federal facility? Yes No |  |  |  |  |  |
| NPDES project or permit tracking number*: |  |  |  |  |  |
| *(This is the unique identifying number assigned to your project by your permitting authority after you have applied for coverage under the appropriate National Pollutant Discharge Elimination System (NPDES) construction general permit.) |  |  |  |  |  |

## 1.2	Contact Information/Responsible Parties

Owner:

Valley Enterprise Investment Company, LLC 5393 East 6850 North

Eden, Utah 84310

(801) 725-1517

Project Manager or Site Supervisor (TBD):

Company:

Name:

Address:

City, State, Zip Code:

Telephone Number:

Fax/Email:

SWPPP Contact (TBD):

Company:

Name:

Address:

City, State, Zip Code:

Telephone Number:

Fax/Email:

This SWPPP was Prepared by:

Great Basin Engineering

Heather Avner

5746 South 1475 East, Suite #200

Ogden, Utah 84403

heathera@greatbasinengineering.com

Phone: (801) 394-4515

Subcontractor(s) (TBD):

Company:

Name:

Address:

City, State, Zip Code:

Telephone Number:

Fax/Email:

Emergency 24-Hour Contact (TBD):

Company:

Name:

Telephone Number:

(Contractor to use additional sheets as necessary)

## 1.3	Nature and Sequence of Construction Activity

This project – the construction of a three lot subdivision – includes the construction of Monaco Drive, a private drive for access the lot as well as utility construction to service the proposed residential structure on the lot.

What is the function of the construction activity?

Residential Commercial Industrial Road Construction Linear Utility

Other (please specify):

Estimated Project Start Date (TBD by Owner): __ __ / __ __ / __ __ __ __

Estimated Project Completion Date (TBD by Owner): __ __ / __ __ / __ __ __ __

## 1.4	Soils, Slopes, Vegetation, and Current Drainage Patterns

The soil in the area is clay and gravel layers. The existing ground water quality at this location is assumed to be that of water which is safe for drinking; the existing storm water quality is typical of storm water flowing from undeveloped hillside. The site is located on a hillside with slopes at approximately 8 horizontal to 1 vertical. The slope is generally from south to north. After construction the slope of the land will be similar to the existing slope with the addition of a road, driveway and retaining walls. The existing site consists of grasses and areas of existing paving with buildings.

## 1.5	Construction Site Estimates

| The following are estimates of the construction site. |  |
| --- | --- |
| Approximate Total project area: | 3.96 acres |
| Construction site area to be disturbed : | 3.96 acres |
| Percentage impervious area before construction: | <5% |
| Runoff coefficient before construction: | <0.19 |
| Percentage impervious area after construction: | 36% |
| Runoff coefficient after construction | 0.42 |

## 1.6	Receiving Waters

A minimal amount of water flows onto the site. A private onsite storm drain system collects all the water Which continues south into an existing drainage channel. From this channel the water makes its way to Pineview Reservoir.

## 1.7	Site Features and Sensitive Areas to be Protected

Not applicable.

## 1.8	Potential Sources of Pollution

The construction of the road on the site may affect the quality of the stormwater discharge from the site. Potential pollutants and sources, other than sediment include and are not limited to Fuels and Oils, Concrete Waste and Soil Erosion. These pollutants could occur throughout the construction site. Also, because the site is located on a hillside, erosion is a potential source of pollution.

## 1.9	Endangered Species Certification

Are endangered or threatened species and critical habitats on or near the project area?

Yes No

According to http://dwrcdc.nr.utah.gov/ucdc/ViewReports/sscounty.pdf, the gray wolf, june sucker an yellow-billed cuckoo which are all considered endangered species have been observed in Weber County. These animals may be found at the construction site. The Contractor should take care in protecting endangered animals if they happen to be found on or near the construction site.

## 1.10	Historic Preservation

Are there any historic sites on or near the construction site?

Yes No

A search of the Utah State Historical Registry yielded no historical sites within 5 miles of the project area.

## 1.11	Applicable Federal, Tribal, State or Local Programs

Not Applicable

## 1.12	Maps

See Appendix B:

Sheet 1 – Storm Water Pollution Prevention Plan

# SECTION 2: EROSION AND SEDIMENT CONTROL BMPS

BMP’s to be used are shown on drawing sheet 1.  Appendix L includes detailed specifications of the selected BMP’s in this section.  Appendix M includes detailed specifications of Non-Selected BMP’s.  It is to the contractors discrepancy to include those Non-Selected BMP’s that he deems necessary.

## 2.1	Minimize Disturbed Area and Protect Natural Features and Soil

- The SWPP areas of disturbance are shown on drawing sheet 1.  Also shown on this sheet are the BMP’s that will be used to limit disturbance outside of the project site.  A silt fence will be installed to prevent sediment from flowing off the site.  Check dams and straw bale barriers will be used to maintain the swales and to prevent erosion.

## 2.2	Phase Construction Activity

Construction will be phased to minimize the impact to the site and surrounding areas as well as to prevent erosion and storm water runoff.

- Phase I – Clear & Grub Existing Vegetation

- BMP’s to be used during Phase I

  - Construction Road Stabilization (CR)

  - Equipment & Vehicle Wash Down Area (EVWA)

  - Stabilized Construction Entrance (SCE)

  - Silt Fence (SF)

  - Vehicle & Equipment Cleaning (VEC)

  - Vehicle & Equipment Fueling (VEF)

- Phase II – Utility Construction Phase

- BMP’s to be used during Phase II

  - Check Dams (CD)

  - Construction Road Stabilization (CR)

  - Equipment & Vehicle Wash Down Area (EVWA)

  - Stabilized Construction Entrance (SCE)

  - Silt Fence (SF)

  - Seeding & Planting (SP)

  - Straw Bale Barrier (STB

  - Vehicle & Equipment Cleaning (VEC)

  - Vehicle & Equipment Fueling (VEF)

- Phase III – Finish Grading Phase

- BMP’s to be used during Phase IV

  - Check Dams (CD)

  - Construction Road Stabilization (CR)

  - Equipment & Vehicle Wash Down Area (EVWA)

  - Stabilized Construction Entrance (SCE)

  - Silt Fence (SF)

  - Seeding & Planting (SP)

  - Straw Bale Barrier (STB)

  - Vehicle & Equipment Cleaning (VEC)

  - Vehicle & Equipment Fueling (VEF)

## 2.3	Control Stormwater Flowing onto and through the Project

Stormwater run-on flowing onto the project site will be treated with the same BMP’s as stormwater falling directly on the site.  Stormwater runon is not anticipated to be very concentrated or excessive in general.

## Stabilize Soils

| BMP Description: Seeding and Planting (SP) |  |
| --- | --- |
| Recommended maintenance: | Shrubs and trees must be adequately watered and fertilized and if needed pruned. Grasses may need to be watered and mowed. |
| Schedule for Maintenance and Inspection: | As necessary. |
| Responsible Staff: | Contractor |

## 2.5	Protect Slopes

| BMP Description: Seeding and Planting (SP) |  |
| --- | --- |
| Recommended maintenance: | Shrubs and trees must be adequately watered and fertilized and if needed pruned. Grasses may need to be watered and mowed. |
| Schedule for Maintenance and Inspection: | As necessary. |
| Responsible Staff: | Contractor |

## 2.6	Protect Storm Drain Inlets

Not applicable

## 2.7	Establish Perimeter Controls and Sediment Barriers

| BMP Description: Silt Fence (SF) |  |
| --- | --- |
| Recommended Maintenance: | Look for runoff bypassing ends of barriers or undercutting barriers. Repair or replace damaged areas of the barrier and remove accumulated sediment. Reanchor fence as necessary to prevent shortcutting. Remove accumulated sediment when it reaches ½ the height of the fence. |
| Schedule of Maintenance and Inspection: | Inspect immediately after any rainfall and at least daily during prolonged rainfall. Inspect weekly during periods of less frequent or no rain. |
| Responsible Staff: | Contractor |

| BMP Description: Straw Bale Barrier (STB) |  |
| --- | --- |
| Recommended Maintenance: | Look for runoff bypassing ends of barriers or undercutting barriers. Repair or replace damaged areas of the barrier and remove accumulated sediment. Realign bales as necessary to provide continuous barrier and fill gaps. Recompact soil around barrier as necessary to prevent piping. |
| Schedule of Maintenance and Inspection: | Inspect immediately after any rainfall and at least daily during prolonged rainfall. Inspect weekly during periods of less frequent or no rain. |
| Responsible Staff: | Contractor |

## 2.8	Retain Sediment On-Site

| BMP Description: Equipment & Vehicle Wash Down Area (EVWA) |  |
| --- | --- |
| Recommended Maintenance: | Repair area and replace gravel as required to maintain control in good working condition. Expand stabilized area as required to accommodate activities. Maintain silt fence as outlined in specific silt fence BMP information sheet. |
| Schedule of Maintenance and Inspection: | Inspect daily for loss of gravel or sediment buildup. Inspect adjacent area for sediment deposit and install additional controls as necessary. |
| Responsible Staff: | Contractor |

## 2.9	Establish Stabilized Construction Exits

| BMP Description: Construction Road Stabilization (CR) |  |
| --- | --- |
| Recommended Maintenance: | Place additional gravel as needed and repair any damaged areas. Maintain any roadside drainage controls. |
| Schedule of Maintenance and Inspection: | Inspect after major rainfall events and at least monthly. |
| Responsible Staff: | Contractor |

| BMP Description: Stabilized Construction Entrance (SCE) |  |
| --- | --- |
| Recommended Maintenance: | Repair entrance and replace gravel as required to maintain control in good working condition. Expand stabilized area as required to accommodate traffic and prevent erosion at driveways. |
| Schedule of Maintenance and Inspection: | Inspect daily for loss of gravel or sediment buildup. Inspect adjacent roadway for sediment deposit and clean by sweeping or shoveling. |
| Responsible Staff: | Contractor |

# SECTION 3: GOOD HOUSEKEEPING BMPS

## 3.1	Material Handling and Waste Management

- All pollutants, including waste materials and demolition debris, that occur on-site during construction will be handled in a way that does not contaminate storm water.

- All chemicals including liquid products, petroleum products, water treatment chemicals, and wastes stored on site will be covered and contained and protected from vandalism.

- Maintenance and repair of all equipment and vehicles involving oil changes, hydraulic system drain down, de-greasing operations, fuel tank drain down and removal, and other activities which may result in the accidental release of contaminants, will be conducted under cover during wet weather and on an impervious surface to prevent the release of contaminants onto the ground.  Materials spilled during maintenance operations will be cleaned up immediately and properly disposed of.

- Wheel wash water will be settled and discharged on site by infiltration.  Wheel wash water will not be discharged to the storm water system or the storm water treatment system.

- Application of agricultural chemicals, including fertilizers and pesticides, will be conducted in a manner and at application rates that will not result in loss of chemical to storm water runoff. Manufacturers’ recommendations will be followed for application rates and procedures.

- pH-modifying sources will be managed to prevent contamination of runoff and storm water collected on site.  The most common sources of pH-modifying materials are bulk cement, cement kiln dust (CKD), fly ash, new concrete washing and curing waters, waste streams generated from concrete grinding and sawing, exposed aggregate processes, and concrete pumping and mixer washout waters.

| BMP Description: Hazardous Waste Management (HWM) |  |
| --- | --- |
| Recommended Maintenance: | Arrange for hazardous waste collection. |
| Recommended Schedule of Maintenance and Inspection: | Inspect hazardous waste receptacles and areas regularly. |
| Responsible Staff: | Contractor |

## 3.2	Establish Proper Building Material Staging Areas

Not applicable

## 3.3	Designate Washout Areas

| BMP Description: Concrete Waste Management (CWM) |  |
| --- | --- |
| Recommended Maintenance: | Inspect subcontractors to ensure that concrete wastes are being properly managed. |
| Recommended Schedule of Maintenance and Inspection: | As necessary, or if using a temporary pit, dispose hardened concrete on a regular basis. |
| Responsible Staff: | Contractor |

## 3.4	Establish Proper Equipment/Vehicle Fueling and Maintenance Practices

| BMP Description: Vehicle & Equipment Cleaning (VEC) |  |
| --- | --- |
| Recommended Maintenance: | Some berm repair may be necessary. |
| Recommended Schedule of Maintenance and Inspection: | As necessary. |
| Responsible Staff: | Contractor |

## 3.5	Control Equipment/Vehicle Washing

| BMP Description: Vehicle & Equipment Fueling (VEF) |  |
| --- | --- |
| Recommended Maintenance: | Keep ample supplies of spill cleanup materials on-site. Inspect fueling areas and storage tanks on a regular schedule. |
| Recommended Schedule of Maintenance and Inspection: | As necessary. |
| Responsible Staff: | Contractor |

## 3.6	Spill Prevention and Control Plan

- The site superintendent will be notified immediately when a spill, or the threat of a spill, is observed.  The superintendent will assess the situation and determine the appropriate response.

- If spills represent an imminent threat of escaping ESC facilities and entering the receiving waters, facility personnel will respond immediately to contain the release and notify the superintendent after the situation has been stabilized.

- Spill kits containing materials and equipment for spill response and cleanup will be maintained at the site.  Each spill kit may contain:

- Oil absorbent pads (one bale)

- Oil absorbent booms (40 feet)

- 55-gallon drums (2)

- 9-mil plastic bags (10)

- Personal protective equipment including gloves and goggles

- If an oil sheen is observed on surface water (e.g., settling ponds, detention pond, swales), absorbent pads and/or booms will be applied to contain and remove the oil.  The source of the oil sheen will also be identified and removed or repaired as necessary to prevent further releases.

- The site superintendent, or his designee, will be responsible for completing the spill reporting form and for reporting the spill to the appropriate state or local agency (see Forms at the end of this section).

- Facility personnel with primary responsibility for spill response and cleanup will receive training from the site superintendent.  This training will include identifying the location of spill kits and other spill response equipment and the use of spill response materials.

- Spill response equipment will be inspected and maintained as necessary to replace any materials used in spill response activities.

- Any spill of oil which 1) violates water quality standards, 2) produces a “sheen” on a surface water, or 3) causes a sludge or emulsion must be reported immediately by telephone to the National Response Center Hotline at (800) 424-8802.

- Any oil, hazardous substance, or hazardous waste release which exceeds the reportable quantity must be reported immediately by telephone to the National Response Center Hotline at (800) 424-8802.

- Any spill of oil or hazardous substance to waters of the state must be reported immediately by telephone to the State of Utah Department of Environmental Quality, Rand Fisher at (801) 533-6065.

- Any release of a hazardous substance that may be a threat to human health or the environment must be reported to the State of Utah Department of Environmental Quality, Rand Fisher at (801) 533-6065.

| BMP Description: Spill Clean-Up (SCU) |  |
| --- | --- |
| Recommended Maintenance: | Use as little water as possible. NEVER HOSE DOWN OR BURY SPILL CONTAMINATED MATERIAL. Use rags or absorbent material for clean-up. Excavate contaminated soils. Dispose of clean-up material and soil as hazardous waste. Document all spills with date, location, substance, volume, actions taken, and other pertinent data. Contact local Fire Department and Sate Division of Environmental Response and Remediation ((801) 536-4100) for any spill of reportable quantity. |
| Schedule of Maintenance and Inspection: | Clean-up spills/leaks immediately and remediate cause. |
| Responsible Staff: | Contractor |

## 3.7	Any Additional BMPs

| BMP Description: Portable Toilets (PT) |  |
| --- | --- |
| Recommended Maintenance: | Regular waste collection should be arranged with licensed service. All waste should be deposited in sanitary sewer system for treatment with appropriate agency approval. |
| Schedule of Maintenance and Inspection: | Portable toilets should be maintained in good working order by licensed service with daily observation for leak protection. |
| Responsible Staff: | Contractor |

## 3.8	Allowable Non-Stormwater Discharge Management

Not applicable

# SECTION 4: SELECTING POST-CONSTRUCTION BMPs

| BMP Description: Spill Clean-Up (SCU) |  |
| --- | --- |
| Recommended Maintenance: | Use as little water as possible. NEVER HOSE DOWN OR BURY SPILL CONTAMINATED MATERIAL. Use rags or absorbent material for clean-up. Excavate contaminated soils. Dispose of clean-up material and soil as hazardous waste. Document all spills with date, location, substance, volume, actions taken, and other pertinent data. Contact local Fire Department and Sate Division of Environmental Response and Remediation ((801) 536-4100) for any spill of reportable quantity. |
| Schedule of Maintenance and Inspection: | Clean-up spills/leaks immediately and remediate cause. |
| Responsible Staff: | Contractor |

| BMP Description: Seeding and Planting (SP) |  |
| --- | --- |
| Recommended maintenance: | Shrubs and trees must be adequately watered and fertilized and if needed pruned. Grasses may need to be watered and mowed. |
| Schedule for Maintenance and Inspection: | As necessary. |
| Responsible Staff: | Contractor |

# SECTION 5: INSPECTIONS

## 5.1	Inspections

1. Inspection Personnel: To be selected by Contractor Company or Organization Name:. Name:. Position:. Address:. City, State, Zip Code:. Telephone Number:. Fax/Email:.

2. Inspection Schedule and Procedures: To be determined by Contractor See Appendix E – Inspection Reports

## 5.2	Delegation of Authority

See Appendix K – Delegation of Authority Duly Authorized Representative(s) for purposes of signing inspection reports, certifications, or other information (by Contractor if Applicable):

Company or Organization Name:

Name:

Position:

Address:

City, State, Zip Code:

Telephone Number:

Fax/Email:

## 5.3	Corrective Action Log

See Appendix F – Corrective Action Log

SECTION 6: RECORDKEEPING AND TRAINING

## 6.1	Recordkeeping

Records of the following will be retained for a minimum period of at least 3 years after the permit is terminated (see attached log):

- Date(s) when major grading activities occur - Date(s) when construction activities temporarily or permanently cease on a portion of the site - Date(s) when an area is either temporarily or permanently stabilized

## 6.2	Log of Changes to the SWPPP

Changes to SWPPP documents and plans should be documented (see Appendix G – SWPPP Amendment Log).

## 6.3	Training

Contractor is responsible for:

- General stormwater and BMP awareness training for staff and subcontractors

- Detailed training for staff and subcontractors with specific stormwater responsibilities

See Appendix J – Training Log

# SECTION 7: FINAL STABILIZATION

- After construction has been completed, the site shall be swept clean and all waste and leftover materials shall be removed from the site.

- All landscaping and planting areas should be well maintained to prevent erosion.  Avoid over watering of landscaping.

- All paved areas should be swept weekly either by hand or by mechanical means to keep the site clear of dirt, dust, and debris.

- Waste materials on-site should be stored in covered containers which are cleaned out regularly.

# SECTION 8: CERTIFICATION AND NOTIFICATION

I understand that this initial document and all initial attachments were prepared by Great Basin Engineering in accordance with a system designed to assure that qualified personnel properly gathered and evaluated the information submitted.  (See page iv for certification.)

I certify under penalty of law that any revisions to the original document and any revisions to attachments were prepared under my direction or supervision in accordance with a system designed to assure that qualified personnel properly gathered and evaluated the information submitted.  Based on my inquiry of the person or persons who manage the system, or those persons directly responsible for gathering the information, the information submitted is, to the best of my knowledge and belief, true, accurate, and complete.  I am aware that there are significant penalties for submitting false information, including the possibility of fine and imprisonment for knowing violations.

| Contractor Name: |  |  | Title: |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Signature: |  |  |  |  | Date: |  |

# SWPPP APPENDICES

Appendix A – General Location Map

Appendix B – Site Maps

Appendix C – Construction General Permit

Appendix D – NOI and Acknowledgement Letter from State

Appendix E – Inspection Reports

Appendix F – Corrective Action Log

Appendix G – SWPPP Amendment Log

Appendix H – Subcontractor Certifications/Agreements

Appendix I – Grading and Stabilization Activities Log

Appendix J – Training Log

Appendix K – Delegation of Authority

Appendix L – Standards and Specifications for Selected BMP’s

Appendix M – Standards and Specifications for Additional Non-Selected BMP’s

Appendix A – General Location Map

Appendix B – Site Maps

Appendix C – Construction General Permit

Appendix D – NOI and Acknowledgement Letter from State

Appendix E – Inspection Reports

Erosion and Sediment Control Inspection Form

Inspector(s):	Date:

Site Name and Location:

Current Weather Conditions:	Last 24 Hours:

Erosion Prevention

| BMP Designation | OK | Not OK | BMP Condition, Corrective Action, General Notes |
| --- | --- | --- | --- |
| Construction Access Trackout? Street Clean? |  |  |  |
| Soil Stabilization Signs of Erosion: Gullies? Slope Failures? Rills? |  |  |  |
| Slope Protection Plastic Condition? Grass Growing? Hydroseed Condition? Matting? |  |  |  |
| Perimeter Control Clearing Limits Marked? Silt Fences? Swales? |  |  |  |
| Conveyances Stable Ditches? Check Dams Intact? Sand Bags? Slope Drains? |  |  |  |
| TESC Management Revisions Required? |  |  |  |
| Water Management Infiltration System? Clean and Dirty Water Separated? Offsite Water Bypassing? |  |  |  |
| Outlet Protection Stabilized? |  |  |  |

Sediment Prevention

| BMP Designation | OK | Not OK | BMP Condition, Corrective Action, General Notes |
| --- | --- | --- | --- |
| Storm water Detention and Monitoring |  |  |  |
| BMP Maintenance |  |  |  |
| Inlet Protection |  |  |  |
| Dust Control |  |  |  |
| Spill Prevention |  |  |  |
| Condition of Discharge Water |  |  |  |
| Comments: |  |  |  |

Spill Report Form

| Location: |  |  |
| --- | --- | --- |
|  |  | Date:___________________ Time: |
| Regulatory agencies notified (date, time, person, agency, and how): |  |  |
| Material spilled: |  |  |
| Quantity spilled: |  |  |
| Source: Cause: |  |  |
| Extent of injuries (if any): Adverse environmental impact (if any): Immediate remedial actions taken at time of spill: Measures taken or planned to prevent recurrence: Additional comments: This report prepared by: (Signature) (Date) (Job title) |  |  |

Appendix F – Corrective Action Log

Project Name:

SWPPP Contact:

| Inspection Date | Inspector Name(s) | Description of BMP Deficiency | Corrective Action Needed (including planned date/responsible person) | Date Action Taken/Responsible person |
| --- | --- | --- | --- | --- |

Appendix G –SWPPP Amendment Log

Project Name:

SWPPP Contact:

| Amendment No. | Description of the Amendment | Date of Amendment | Amendment Prepared by [Name(s) and Title] |
| --- | --- | --- | --- |

Appendix H –Subcontractor Certifications/Agreements

SUBCONTRACTOR CERTIFICATION

STORMWATER POLLUTION PREVENTION PLAN

Project Number:

Project Title:

Operator(s):

As a subcontractor, you are required to comply with the Stormwater Pollution Prevention Plan (SWPPP) for any work that you perform on-site.  Any person or group who violates any condition of the SWPPP may be subject to substantial penalties or loss of contract.  You are encouraged to advise each of your employees working on this project of the requirements of the SWPPP.  A copy of the SWPPP is available for your review at the office trailer.

Each subcontractor engaged in activities at the construction site that could impact stormwater must be identified and sign the following certification statement:

I certify under the penalty of law that I have read and understand the terms and conditions of the SWPPP for the above designated project and agree to follow the BMPs and practices described in the SWPPP.

This certification is hereby signed in reference to the above named project:

Company:

Address:

Telephone Number:

Type of construction service to be provided:

Signature:

Title:

Date:

Appendix I –Grading and Stabilization Activities Log

Project Name:

SWPPP Contact:

| Date Grading Activity Initiated | Description of Grading Activity | Date Grading Activity Ceased (Indicate Temporary or Permanent) | Date When Stabilization Measures are Initiated | Description of Stabilization Measure and Location |
| --- | --- | --- | --- | --- |

Appendix J –SWPPP Training Log

Stormwater Pollution Prevention Training Log

Project Name:

Project Location:

Instructor’s Name(s):

Instructor’s Title(s):

Course Location: 	  Date:

Course Length (hours):

Stormwater Training Topic:  (check as appropriate)

|  | Erosion Control BMPs |  | Emergency Procedures |
| --- | --- | --- | --- |
|  | Sediment Control BMPs |  | Good Housekeeping BMPs |
|  | Non-Stormwater BMPs |  |  |

Specific Training Objective:

Attendee Roster:  (attach additional pages as necessary)

| No. | Name of Attendee | Company |
| --- | --- | --- |
| 1 |  |  |
| 2 |  |  |
| 3 |  |  |
| 4 |  |  |
| 5 |  |  |
| 6 |  |  |
| 7 |  |  |
| 8 |  |  |
| 9 |  |  |
| 10 |  |  |

Appendix K –Delegation of Authority Form

Delegation of Authority

I, _______________________ (name), hereby designate the person or specifically described position below to be a duly authorized representative for the purpose of overseeing compliance with environmental requirements, including the Construction General Permit, at the ____________________________________ construction site.  The designee is authorized to sign any reports, stormwater pollution prevention plans and all other documents required by the permit.

________________________________________ (name of person or position)

________________________________________ (company)

________________________________________ (address)

________________________________________ (city, state, zip)

________________________________________ (phone)

By signing this authorization, I confirm that I meet the requirements to make such a designation as set forth in ____________________________________ (Reference State Permit), and that the designee above meets the definition of a “duly authorized representative” as set forth in ____________________________________ (Reference State Permit).

I certify under penalty of law that this document and all attachments were prepared under my direction or supervision in accordance with a system designed to assure that qualified personnel properly gathered and evaluated the information submitted.  Based on my inquiry of the person or persons who manage the system, or those persons directly responsible for gathering the information, the information submitted is, to the best of my knowledge and belief, true, accurate, and complete.  I am aware that there are significant penalties for submitting false information, including the possibility of fine and imprisonment for knowing violations.

Name:

Company:

Title:

Signature:

Date:

Appendix L –Standards and Specifications for Selected BMP’s

Appendix M –Standards and Specifications for Additional Non-Selected BMP’s