A:  SWPPP Template – Utah

Instructions

To help you develop the narrative section of your construction site SWPPP, the DWQ has modified the U.S Environmental Protection Agency (EPA) electronic SWPPP template to fit the needs of NOI applicants in Utah.  The template is designed to help guide you through the SWPPP development process and help ensure that your SWPPP addresses all the necessary elements stated in your construction general permit.  It may be helpful to use this template with EPA’s guidance on Developing Your Storm water Pollution Prevention Plan. Both are available on EPA’s website at www.epa.gov/npdes/swpppguide

This template covers most of the SWPPP elements that the Utah construction general permit requires, however, you are strongly encouraged to customize this template. There are two major reasons to customize this template:

To reflect the terms and conditions of the State construction general permit; and

To reflect the conditions at your site

Using the SWPPP Template

This template is ordered in reference to Section 7, Storm Water Pollution Prevention Plan (SWPPP).

Each section of this template includes “instructions” and space for project information. You should read the instructions for each section before you complete that section. This template was developed in Word so that you can easily add tables and additional text. Some sections may require only a brief description while others may require several pages of explanation.

Tips for completing the SWPPP template

- If there is more than one construction operator for your project, consider coordinating development of your SWPPP with the other operators.

- Multiple operators may share the same SWPPP, but make sure that responsibilities are clearly described.

- Modify this SWPPP template so that it addresses the requirements in your construction general permit and meets the needs of your project. Consider adding permit citations in the SWPPP when you address a specific permit requirement.

Storm Water Pollution Prevention Plan

for:

Jacquelyn Estates

4900 West 2200 South

Taylor, Utah

Insert Project Site Telephone Number (if applicable)

Operator(s):

CJ Homes Inc.

Craig Standing

5337 Ridgedale Dr

Ogden, Utah 84403

801-698-4048

Standing7949@clear.net

SWPPP Contact(s):

CJ Homes Inc.

Craig Standing

5337 Ridgedale Dr

Ogden, Utah 84403

801-698-4048

Standing7949@clear.net

SWPPP Preparation Date:

3/22/2015

Estimated Project Dates:

Project Start Date:  04/30/2015

Project Completion Date: 06/30/2018

Contents

SECTION 1: CONTACT INFORMATION/ RESPONSIBLE PARTIES	1

1.1	Owner(s), Operator, Contractors	1

1.2	Storm Water Team	2

SECTION 2: SITE EVALUATION, ASSESSMENT, & PLANNING	4

2.1	Project/Site Information	4

2.2	Nature of Construction Activity	5

2.3	Construction Site Estimates	5

2.4	Soils, Slopes, Vegetation, and Current Drainage Patterns	6

2.5	Emergency Related Projects	6

2.6	Phase/Sequence of Construction Activity	7

2.7	Site Features and Sensitive Areas to be Protected	8

2.8	Maps	8

SECTION 3: POLLUTION PREVENTION STANDARDS	9

3.1	Potential Sources of Pollution	9

3.2	Non-Storm Water Discharges	10

3.3	Natural Buffers or Equivalent Sediment Controls	11

SECTION 4: EROSION AND SEDIMENT CONTROLS	15

4.1	Minimize Disturbed Area and Protect Natural Features and Soil	16

4.2	Establish Perimeter Controls and Sediment Barriers	16

4.3	Retain Sediment On-Site	17

4.4	Establish Stabilized Construction Exits	17

4.5	Protect Slopes	18

4.6	Stockpiled Sediment or Soil	19

4.7	Minimize Dust	19

4.8	Topsoil	20

4.9	Soil Compaction	20

4.10	High Altitude/Heavy Snows	21

4.11	Linear Activities	22

4.12	Chemical Treatment	22

4.13	Stabilize Soils	23

4.14	Final Stabilization	24

SECTION 5: POLLUTION PREVENTION	25

5.1	Spill Prevention and Response	25

5.2	Construction and Domestic Waste	26

Repeat as needed	26

5.3	Washing of Applicators and Containers used for Concrete, Paint or Other Materials	27

5.4	Establish Proper Building Material Staging Areas	27

5.5	Establish Proper Equipment/Vehicle Fueling and Maintenance Practices	28

5.6	Control Equipment/Vehicle Washing	29

5.7	Pesticides, Herbicides, Insecticides, Fertilizers, and Landscape Materials	29

5.8	Other Pollution Prevention Practices	30

SECTION 6: INSPECTIONS & CORRECTIVE ACTIONS	31

6.1	Inspections	31

Reduction in Inspection Frequency (if applicable)	32

6.2	Corrective Actions	32

6.3	Delegation of Authority	32

SECTION 7: TRAINING AND RECORDKEEPING	34

7.1	Training	34

7.2	Recordkeeping	35

7.3	Log of Changes to the SWPPP	35

SECTION 8: WATER QUALITY	36

8.1	UIC Class 5 Injection Wells	36

8.2	Discharge Information	37

8.3	Receiving Waters	37

8.4	Impaired Waters	38

8.5	High Water Quality	38

8.6	Dewatering Practices	38

8.7	Control Storm Water Flowing onto and through the Project	39

8.8	Protect Storm Drain Inlets	40

SECTION 9: POST-CONSTRUCTION BMPs	41

SECTION 10: CERTIFICATION	42

SWPPP APPENDICES	43

Appendix A – General Location Map	43

Appendix B – Site Maps	43

Appendix C – Construction General Permit	43

Appendix D – NOI and Acknowledgement Letter from EPA/State/MS4	43

Appendix E – Inspection Reports	43

Appendix F – Corrective Action Log (or in Part 5.3)	43

Appendix G – SWPPP Amendment Log (or in Part 7.4)	43

Appendix H – Subcontractor Certifications/Agreements	43

Appendix I – Grading and Stabilization Activities Log	43

Appendix J – Training Log	43

Appendix K – Delegation of Authority	43

Appendix L – Additional Information (i.e. Documentation; other permits such as dewatering, stream alteration, wetland; and out of date swppp documents)	43

Appendix M – BMP Specifications	43

# SECTION 1: CONTACT INFORMATION/ RESPONSIBLE PARTIES

## 1.1	Owner(s), Operator, Contractors

Instructions:

List the operator(s), project managers, storm water contact(s), and person or organization that prepared the SWPPP.  Indicate respective responsibilities, where appropriate.

Also, list subcontractors expected to work on-site. Notify subcontractors of storm water requirements applicable to their work.

See SWPPP Guide, Chapter 2.B.

Owner(s):

CJ Homes Inc. Craig Standing 5337 Ridgedale Dr Ogden, Utah 84403 801-698-4048 Standing7949@gmail.com

Operator(s) & Project Manager(s):

CJ Homes Inc. Craig Standing 5337 Ridgedale Dr Ogden, Utah 84403 801-698-4048 Standing7949@gmail.com

Site Supervisor(s): CJ Homes Inc. Craig Standing 5337 Ridgedale Dr Ogden, Utah 84403 801-698-4048 Standing7949@gmail.com

SWPPP Contact(s):

CJ Homes Inc.

Craig Standing

5337 Ridgedale Dr

Ogden, Utah 84403

801-698-4048

Standing7949@gmail.com

CJ Homes Inc.

Subcontractor(s):

Rock Hard Excavation Ray Holmes 2841 W 1800 S West Haven, Utah 84401 801 430-0670

Emergency 24-Hour Contact: CJ Homes Inc. Craig Standing 801 698-4048

## 1.2	Storm Water Team

Instructions (see CGP Part 7.2.1):

Identify the staff members (by name or position) that comprise the project’s storm water team as well as their individual responsibilities.  At a minimum the storm water team is comprised of individuals who are responsible for overseeing the development of the SWPPP, any later modifications to it, and for compliance with the requirements in this permit (i.e., installing and maintaining storm water controls, conducting site inspections, and taking corrective actions where required).

Each member of the storm water team must have ready access to either an electronic or paper copy of applicable portions of the 2014 UCGP and your SWPPP.

Owner/Developer

Developer

Craig Standing

801 698-4048

Standing7949@clear.net

Excavation/infrastructure

Excavator: Ray Holmes

801 430-0670

# SECTION 2: SITE EVALUATION, ASSESSMENT, & PLANNING

## 2.1	Project/Site Information

Instructions:

In this section, you can gather some basic site information that will be helpful to you later when you file for permit coverage.

For more information, see Developing Your Storm Water Pollution Prevention Plan: A SWPPP Guide for Construction Sites (also known as the SWPPP Guide), Chapter 2

Detailed information on determining your site’s latitude and longitude can be found at www.epa.gov/npdes/stormwater/latlong

| Project/Site Name: Jacquelyn Estates |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Project Street/Location: 4900 West 2200 South |  |  |  |  |  |
| City: Taylor |  | State: Utah |  | ZIP Code: |  |
| County or Similar Subdivision: Weber |  |  |  |  |  |
| Latitude/Longitude (Use one of three possible formats, and specify method) |  |  |  |  |  |
| Latitude: | Longitude: |  |  |  |  |
| 1. 41 º 13 ' 41.34'' N (degrees, minutes, seconds) | 1. 112º 05 ' 48.46'' W (degrees, minutes, seconds) |  |  |  |  |
| 2. _ _ º _ _ . _ _' N (degrees, minutes, decimal) | 2. _ _ º _ _ . _ _' W (degrees, minutes, decimal) |  |  |  |  |
| 3. _ _ . _ _ _ _ º N (decimal) | 3. _ _ . _ _ _ _ º W (decimal) |  |  |  |  |
| Method for determining latitude/longitude: |  |  |  |  |  |
| USGS topographic map (specify scale: ) |  |  | EPA Web site |  | xGPS |
| Other (please specify): |  |  |  |  |  |
| Is the project located in Indian country? Yes x No |  |  |  |  |  |
| If yes, name of Reservation, or if not part of a Reservation, indicate "not applicable." |  |  |  |  |  |
| Is this project considered a federal facility? Yes x No |  |  |  |  |  |
| UPDES project or permit tracking number*: |  |  |  |  |  |
| *(This is the unique identifying number assigned to your project by your permitting authority after you have applied for coverage under the appropriate National Pollutant Discharge Elimination System (UPDES) construction general permit.) |  |  |  |  |  |

## 2.2	Nature of Construction Activity

Instructions:

Briefly describe the nature of the construction activity and approximate time frames (one or more paragraphs, depending on the nature and complexity of the project).

For more information, see SWPPP Guide, Chapter 3.A.

Describe the general scope of the work for the project, major phases of construction, etc.:

INSERT TEXT HERE

What is the function of the construction activity?

x Residential Commercial Industrial Road Construction Linear Utility

Other (please specify):

Estimated Project Start Date: _04_ __ / _30_ __ / __2015 __ __ __

Estimated Project Completion Date: __06 __ / __ 30__ / _2018_ __ __ __

## 2.3	Construction Site Estimates

Instructions:

Estimate the area to be disturbed by excavation, grading, or other construction activities, including dedicated off-site borrow and fill areas.

Calculate the percentage of impervious surface area before and after construction

Calculate the runoff coefficients before and after construction.

For more information, see SWPPP Guide, Chapter 3.A and Appendix C.

| The following are estimates of the construction site. |  |
| --- | --- |
| Total project area: | 14.0 acres |
| Construction site area to be disturbed : | 3 acres |
| Percentage impervious area before construction: | 0% |
| Runoff coefficient before construction: |  |
| Percentage impervious area after construction: | 3.5% |
| Runoff coefficient after construction |  |

## 2.4	Soils, Slopes, Vegetation, and Current Drainage Patterns

Instructions:

Describe the existing soil conditions at the construction site including soil types, slopes and slope lengths, drainage patterns, and other topographic features that might affect erosion and sediment control.

Also, note any historic site contamination evident from existing site features and known past usage of the site.

This information should also be included on your site maps (See SWPPP Guide, Chapter 3.C.).

For more information, see SWPPP Guide, Chapter 3.A.

Soil type(s): topsoil/sandy loam

Slopes (describe current slopes and note any changes due to grading or fill activities): west

Drainage Patterns (describe current drainage patterns and note any changes dues to grading or fill activities): Drain to the west of the property/natural slope

Vegetation: native grass/natural vegetation

Other:

Instructions:

See Part 1.21. in the UCGP.  To be an emergency related project is must be considered a public emergency and the cause must be documented along with the description of necessary construction to reestablish effected public services.

2.5	Emergency Related Projects

Emergency-Related Project? Yes x No

Response to a public emergency (see Part 1.2.1); natural disaster, extreme flooding conditions, etc.

PROVIDE INFORMATION SUTSTANTIATING ITS OCCURRENCE

INSERT DESCRITPION OF CONSTRUCTION THAT WAS NECESSARY TO REESTABLISH EFFECTED PUBLIC SERVICES

Instructions:

Describe the intended construction sequencing and timing of major activities, including any opportunities for phasing grading and stabilization activities to minimize the overall amount of disturbed soil that will be subject to potential erosion at one time.  Also, describe opportunities for timing grading and stabilization so that all or a majority of the soil disturbance occurs during a time of year with less erosion potential (i.e., during the dry or less windy season). (For more information, see SWPPP Guide, Chapter 4, ESC Principle 2.) It might be useful to develop a separate, detailed site map for each phase of construction.

See UCGP Section 7.2.4 for detailed information.

Also, see EPA’s Construction Sequencing BMP Fact Sheet at http://www.epa.gov/npdes/stormwater/menuofbmps/construction/cons_seq)

2.6	Phase/Sequence of Construction Activity

Phase I

- Describe Phase:   Phase one will consist of six residential building lots with supportive infrastructure. Land excavation and disturbance will be all at once per provided SWPP Plan Drawing

- Duration of phase (4/30/2015- 6/30/2015)

- BMP:  Silt fencing will be installed at any downgrade location on the site where it is determined that flows from the site would impact an existing drainage irrigation or neighboring property

- Describe stabilization methods:  A filtering method (straw bales, filtering fabric or an approved equal) will be in any drainage that may be impacted by the construction.  The filtering will be installed to intercept the flows away from the site and in the vicinity of the site where it is determined that the construction on the site would damage or degrade the quality of water flow.   A clean up area will be established at the site for removal of mud or other debris that accumulates on the construction equipment prior to leaving the site.  The cleanup area shall be graded to prevent any contamination to adjacent properties, streets, and water ways by silt, mud, or other pollutants.

Phase II

- Describe Phase:   Phase two will consist of 13 residential building lots with supportive infrastructure. Land excavation and disturbance will be all at once per provided SWPP Plan drawing.

- Duration of phase (4/30/2015- 6/30/2015)

- BMP:  Silt fencing will be installed at any downgrade location on the site where it is determined that flows from the site would impact an existing drainage irrigation or neighboring property

- Describe stabilization methods:  A filtering method (straw bales, filtering fabric or an approved equal) will be in any drainage that may be impacted by the construction.  The filtering will be installed to intercept the flows away from the site and in the vicinity of the site where it is determined that the construction on the site would damage or degrade the quality of water flow.   A clean up area will be established at the site for removal of mud or other debris that accumulates on the construction equipment prior to leaving the site.  The cleanup area shall be graded to prevent any contamination to adjacent properties, streets, and water ways by silt, mud, or other pollutants.

## 2.7	Site Features and Sensitive Areas to be Protected

Instructions:

Describe unique site features including streams, stream buffers, wetlands, specimen trees, natural vegetation, steep slopes, or highly erodible soils that are to be preserved.

Describe measures to protect these features.

Include these features and areas on your site maps.

This permit does not diminish from or alter in any way a permittees responsibility under the Endangered Species Act (EAS).  This permit does not have any requirements pertaining to the ESA.  UTRC00000 CGP 1.1.5.

This permit does not diminish from or alter in any way a permittees responsibility under the National Historic Preservation Act (NHPA).  This permit does not have any requirements pertaining to the NHPA.  UTRC00000 CGP 1.1.6.

For more information, see SWPPP Guide, Chapter 3.A and 3.B.

The property site has no unique streams, wetland specimens etc. since Weber County has already approved this project.  This land has been in greenbelt for the past 50 years and has been used for growing alfalfa feed for livestock.

## 2.8	Maps

Instructions:

Attach site maps.  For most projects, a series of site maps is recommended.  The first should show the undeveloped site and its current features.  An additional map or maps should be created to show the developed site or for more complicated sites show the major phases of development.

These maps should include the following:

Direction(s) of storm water flow and approximate slopes before and after major grading activities;

Areas and timing of soil disturbance;

Areas that will not be disturbed;

Natural features to be preserved;

Locations of major structural and non-structural BMPs identified in the SWPPP;

Locations and timing of stabilization measures;

Locations of off-site material, waste, borrow, or equipment storage areas;

Locations of all waters of the United States, including wetlands;

Locations where storm water discharges to a surface water;

Locations of storm drain inlets; and

Areas where final stabilization has been accomplished.

For more information, see SWPPP Guide, Chapter 3.C.

Include the site maps with the SWPPP (Appendix A).

# SECTION 3: POLLUTION PREVENTION STANDARDS

Instructions:

Describe the key good housekeeping and pollution prevention (P2) BMPs that will be implemented to control pollutants in storm water (UCGP Part 2.3).

For more information, see SWPPP Guide, Chapter 5.

Consult your states or local jurisdiction’s design manual or resources in Appendix D of the SWPPP Guide.

For more information or ideas on BMPs, see EPA’s National Menu of BMPs http://www.epa.gov/npdes/stormwater/menuofbmps

- A filtering method (straw bales, filtering fabric or an approved equal) will be in any drainage that may be impacted by the construction.  The filtering will be installed to intercept the flows away from the site and in the vicinity of the site where it is determined that the construction on the site would damage or degrade the quality of water flow.   A clean up area will be established at the site for removal of mud or other debris that accumulates on the construction equipment prior to leaving the site.  The cleanup area shall be graded to prevent any contamination to adjacent properties, streets, and water ways by silt, mud, or other pollutants.

## 3.1	Potential Sources of Pollution

Instructions:

Identify and list all potential sources of sediment, which may reasonably be expected to affect the quality of storm water discharges from the construction site.

Identify and describe all potential sources of pollution or pollutant-generating activity (e.g., paving operations; concrete, paint, and stucco washout and waste disposal; solid waste storage and disposal), other than sediment, which could be exposed to rainfall or snowmelt, and may reasonably be expected to discharges from the construction site.

For more information, see SWPPP Guide, Chapter 3.A.

Potential sources of sediment to storm water runoff: Grubbing or clearing of native soil. Parcel will be graded to direct storm and irrigation water flows into drainage easements and street improvements and not to impact downstream properties

Potential pollutants and sources, other than sediment, to storm water runoff:

INSERT TEXT OR USE TABLE BELOW

| Pollutant-Generating Activity | Pollutants or Pollutant Constituents (that could be discharged if exposed to storm water) | Location on Site (or reference SWPPP site map where this is shown) |
| --- | --- | --- |
| Grading roads | gravel | SWPPP storage area |
| Finish roads | asphalt | SWPPP storage area |
| Curb and gutter | Concrete dust | Road perimeter SWPPP Map |
| Solid waste from construction | Wood fibers | Lot locations/solid storage |
| Driveways/cutting curbs | Concrete dust | Lot locations/SWPPP Map |

Include additional rows as necessary.

## 3.2	Non-Storm Water Discharges

Instructions:

Identify all allowable sources of non-storm water discharges that are not previously identified. UCGP Part 7.2.9

The allowable non-storm water discharges identified might include the following (see your permit for an exact list):

- Waters used to wash vehicles where detergents are not used

- Water used to control dust

- Potable water including uncontaminated water line flushings

- Routine external building wash down that does not use detergents

- Pavement wash waters where spills or leaks of toxic or hazardous materials have not occurred (unless all spilled material has been removed) and where detergents are not used

- Uncontaminated air conditioning or compressor condensate

- Uncontaminated ground water or spring water

- Foundation or footing drains where flows are not contaminated with process materials such as solvents

- Uncontaminated excavation dewatering

- Landscape irrigation

Identify measures used to eliminate or reduce these discharges and the BMPs used to prevent them from becoming contaminated.

For more information, see SWPPP Guide, Chapter 3.A.

List allowable non-storm water discharges and the measures used to eliminate or reduce them and to prevent them from becoming contaminated:

| Authorized Non-Storm Water Discharges | Comments |
| --- | --- |
| Water used to wash vehicles | Cleanup area graded to prevent contamination |
| Water used to compact road | Control method of discharge/vehicle flow meter |

Include additional rows as necessary.

| BMP Description: clean tires before leaving site |  |
| --- | --- |
| Installation Schedule: | Daily/or as required |
| Maintenance and Inspection: | weekly |
| Responsible Staff: | CJ Homes Inc. |

| BMP Description: Maintain speeds |  |
| --- | --- |
| Installation Schedule: | daily |
| Maintenance and Inspection: | weekly |
| Responsible Staff: | CJ Homes Inc. |

Repeat as needed

## 3.3	Natural Buffers or Equivalent Sediment Controls

Instructions (see CGP Parts 2.1.2.1 and 7.2.9, and Appendix G):

- This section only applies to you if a surface water is located within 50 feet your construction activities.  If this is the case, consult CGP Part 2.1.2.a and Appendix D for information on how to comply with the buffer requirements.

Describe the compliance alternative (CGP Part 2.1.2.a.i, ii, iii, or iv) that was chosen to meet the buffer requirements, and include any required documentation supporting the alternative selected.  The compliance alternative selected must be maintained throughout the duration of permit coverage.  However, if you select a different compliance alternative during your period of permit coverage, you must modify your SWPPP to reflect this change.

If you qualify for one of the exceptions in CGP Part 2.1.2.a.v, include documentation related to your qualification for such exceptions.

Buffer Compliance Alternatives

Are there any surface waters within 50 feet of your project’s earth disturbances?      YES      x NO

(Note:  If no, no further documentation is required for the SWPPP Template.)

Check the compliance alternative that you have chosen:

x  I will provide and maintain a 50-foot undisturbed natural buffer.

(Note (1):  You must show the 50-foot boundary line of the natural buffer on your site map.)

(Note (2):  You must show on your site map how all discharges from your construction disturbances through the natural buffer area will first be treated by the site’s erosion and sediment controls.  Also, show on the site map any velocity dissipation devices used to prevent erosion within the natural buffer area.)

x  I will provide and maintain an undisturbed natural buffer that is less than 50 feet and is supplemented by additional erosion and sediment controls, which in combination achieves the sediment load reduction equivalent to a 50-foot undisturbed natural buffer.

(Note (1):  You must show the boundary line of the natural buffer on your site map.)

(Note (2):  You must show on your site map how all discharges from your construction disturbances through the natural buffer area will first be treated by the site’s erosion and sediment controls.  Also, show on the site map any velocity dissipation devices used to prevent erosion within the natural buffer area.)

- INSERT WIDTH OF NATURAL BUFFER TO BE RETAINED

- INSERT EITHER ONE OF THE FOLLOWING:

(1) THE ESTIMATED SEDIMENT REMOVAL FROM A 50-FOOT BUFFER USING APPLICABLE INFORMATION IN APP. D, 2.2.2. INCLUDE INFORMATION ABOUT THE BUFFER VEGETATION AND SOIL TYPE THAT PREDOMINATE AT YOUR SITE

OR

(2) IF YOU CONDUCTED A SITE-SPECIFIC CALCULATION FOR THE ESTIMATED SEDIMENT REMOVAL OF A 50-FOOT BUFFER, PROVIDE THE SPECIFIC REMOVAL EFFICIENCY, AND INFORMATION YOU RELIED UPON TO MAKE YOUR SITE-SPECIFIC CALCULATION.

- INSERT DESCRIPTION OF ADDITIONAL EROSION AND SEDIMENT CONTROLS TO BE USED IN COMBINATION WITH NATURAL BUFFER AREA

- INSERT THE FOLLOWING INFORMATION:

- (1) SPECIFY THE MODEL OR OTHER TOOL USED TO ESTIMATE SEDIMENT LOAD REDUCTIONS FROM THE COMBINATION OF THE BUFFER AREA AND ADDITIONAL EROSION AND SEDIMENT CONTROLS INSTALLED AT YOUR SITE, AND

- (2) INCLUDE THE RESULTS OF CALCULATIONS SHOWING THAT THE COMBINATION OF YOUR BUFFER AREA AND THE ADDITIONAL EROSION AND SEDIMENT CONTROLS INSTALLED AT YOUR SITE WILL MEET OR EXCEED THE SEDIMENT REMOVAL EFFICIENCY OF A 50-FOOT BUFFER

It is infeasible to provide and maintain an undisturbed natural buffer of any size, therefore I will implement erosion and sediment controls that achieve the sediment load reduction equivalent to a 50-foot undisturbed natural buffer.

- INSERT RATIONALE FOR CONCLUDING THAT IT IS INFEASIBLE TO PROVIDE AND MAINTAIN A NATURAL BUFFER OF ANY SIZE

- INSERT EITHER ONE OF THE FOLLOWING:

OR

(2) IF YOU CONDUCTED A SITE-SPECIFIC CALCULATION FOR THE ESTIMATED SEDIMENT REMOVAL OF A 50-FOOT BUFFER, PROVIDE THE SPECIFIC REMOVAL EFFICIENCY, AND INFORMATION YOU RELIED UPON TO MAKE YOUR SITE-SPECIFIC CALCULATION.

- INSERT DESCRIPTION OF ADDITIONAL EROSION AND SEDIMENT CONTROLS TO BE USED IN COMBINATION WITH NATURAL BUFFER AREA

- INSERT THE FOLLOWING INFORMATION:

- (1) SPECIFY THE MODEL OR OTHER TOOL USED TO ESTIMATE SEDIMENT LOAD REDUCTIONS FROM THE EROSION AND SEDIMENT CONTROLS INSTALLED AT YOUR SITE, AND

- (2) INCLUDE THE RESULTS OF CALCULATIONS SHOWING THAT THE ADDITIONAL EROSION AND SEDIMENT CONTROLS INSTALLED AT YOUR SITE WILL MEET OR EXCEED THE SEDIMENT REMOVAL EFFICIENCY OF A 50-FOOT BUFFER

I qualify for one of the exceptions in Part 2.1.2.a.v.  (If you have checked this box, provide information on the applicable buffer exception that applies, below.)

Buffer Exceptions

Which of the following exceptions to the buffer requirements applies to your site?

x  There is no discharge of storm water to the surface water that is located 50 feet from my construction disturbances.

(Note:  If this exception applies, no further documentation is required for Section 4.1 of the Template.)

No natural buffer exists due to preexisting development disturbances that occurred prior to the initiation of planning for this project.

(Note (1):  If this exception applies, no further documentation is required for Section 2.2 of the Template.)

(Note (2):  Where some natural buffer exists but portions of the area within 50 feet of the surface water are occupied by preexisting development disturbances, you must still comply with the one of the CGP Part 2.1.2.a compliance alternatives.)

For a “linear project” (defined in Appendix A), site constraints (e.g., limited right-of-way) make it infeasible for me to meet any of the CGP Part 2.1.2.a.v.3 compliance alternatives.  Include documentation here of the following:

(1) Why it is infeasible for you to meet one of the buffer compliance alternative, and (2) Buffer width retained and/or supplemental erosion and sediment contorls to treat discharges to the surface water.

The project qualifies as “small residential lot” construction (defined in Part 2.1.2.a.v.3 and in Appendix D).

For Alternative 1 (see Appendix D, Part 2.3.a):

- INSERT WIDTH OF NATURAL BUFFER TO BE RETAINED

- INSERT APPLICABLE REQUIREMENTS BASED ON TABLE D-1

- INSERT DESCRIPTION OF HOW YOU WILL COMPLY WITH THESE REQUIREMENTS

For Alternative 2 (see Appendix D, Part 2.3.b):

- INSERT (1) THE ASSIGNED RISK LEVEL BASED ON APPLICABLE TABLE IN APP. D, PART 2.3.2.b, AND (2) THE PREDOMINANT SOIL TYPE AND AVERAGE SLOPE AT YOUR SITE

- INSERT APPLICABLE REQUIREMENTS BASED ON APP. D, TABLE D-2

- INSERT DESCRIPTION OF HOW YOU WILL COMPLY WITH THESE REQUIREMENTS

Buffer disturbances are authorized under a CWA Section 404 permit.

INSERT DESCRIPTION OF ANY EARTH DISTURBANCES THAT WILL OCCUR WITHIN THE BUFFER AREA

(Note (1):  If this exception applies, no further documentation is required for Section 2.2 of the Template.)

(Note (2):  This exception only applies to the limits of disturbance authorized under the Section 404 permit, and does not apply to any upland portion of the construction project.)

Buffer disturbances will occur for the construction of a water-dependent structure or water access area (e.g., pier, boat ramp, and trail).  INSERT DESCRIPTION OF ANY EARTH DISTURBANCES THAT WILL OCCUR WITHIN THE BUFFER AREA

(Note (1):  If this exception applies, no further documentation is required for Section 2.2 of the Template.)

# SECTION 4: EROSION AND SEDIMENT CONTROLS

Instructions:

See Section 2 in the UCGP.  Describe the erosion and sediment controls (BMPs) that will be implemented to control pollutants in storm water discharges.  For each major activity identified, do the following

- Clearly describe appropriate control measures.

- Describe the general sequence during the construction process in which the measures will be implemented.

- Describe the maintenance and inspection procedures that will be used for that specific BMP.

- Include protocols, thresholds, and schedules for cleaning, repairing, or replacing damaged or failing BMPs.

- Identify staff responsible for maintaining BMPs.

- (If your SWPPP is shared by multiple operators, indicate the operator responsible for each BMP.)

Categorize each BMP under one of the following 10 areas of BMP activity as described below:

4.1	Minimize disturbed area and protect natural features and soil

4.2	Phase Construction Activity

4.3	Control Storm Water flowing onto and through the project

4.4	Stabilize Soils

4.5	Protect Slopes

4.6	Protect Storm Drain Inlets

4.7	Establish Perimeter Controls and Sediment Barriers

4.8	Retain Sediment On-Site and Control Dewatering Practices

4.9	Establish Stabilized Construction Exits

4.10	High Altitude/Heavy Snows

4.11	Linear Activities

4.12	Chemical Treatment

4.13	Stabilize Soils

4.14	Final Stabilization

Note the location of each BMP on your site map(s).

For any structural BMPs, you should provide design specifications and details and refer to them. Attach them as appendices to the SWPPP or within the text of the SWPPP.

For more information, see SWPPP Guide, Chapter 4.

Consult your MS4’s or other local jurisdiction’s design manual or one of those listed in Appendix D of the SWPPP Guide.

For more information or ideas on BMPs, see EPA’s National Menu of BMPs http://www.epa.gov/npdes/stormwater/menuofbmps

- Phase 1:  Silt fencing will be installed at any downgrade location on the site where it is determined that flows from the site would impact an existing drainage, irrigation or neighboring property. Such fencing will be constructed according with weber county requirements and will include a collection swale on the upgrade side of the fencing for collection of sediments.

- A filtering method (straw bales, filtering fabric or an approved equal) will be in any drainage that may be impacted by the construction.  The filtering will be installed to intercept the flows away from the site and in the vicinity of the site where it is determined that the construction on the site would damage or degrade the quality of water flow.

- A clean up area will be established at the site for removal of mud or other debris that accumulates on the construction equipment prior to leaving the site.  The cleanup area shall be graded to prevent any contamination to adjacent properties, streets, and water ways by silt, mud, or other pollutants.

## 4.1	Minimize Disturbed Area and Protect Natural Features and Soil

Instructions:

Describe the areas that will be disturbed with each phase of construction and the methods (e.g., signs, fences) that you will use to protect those areas that should not be disturbed.  Describe natural features identified earlier and how each will be protected during construction activity. Also describe how topsoil will be preserved.  Include these areas and associated BMPs on your site map(s) also.  (For more information, see SWPPP Guide, Chapter 4, ESC Principle 1.)

Also, see EPA’s Preserving Natural Vegetation BMP Fact Sheet at www.epa.gov/npdes/stormwater/menuofbmps/construction/perserve_veg

- Phase1:  About three acres will be disturbed during this phase.  Areas under construction will be protected by silt barrier fences to reduce sediment.  The natural features are the native grasses and alfalfa growth

- Phase 2:  About 6.5 acres will be disturbed in this phase.  Areas under construction will be protected by silt barrier fences to reduce sediment.  The natural features are the native grasses and alfalfa growth

## 4.2	Establish Perimeter Controls and Sediment Barriers

Instructions:

Describe structural practices (e.g., silt fences or fiber rolls) including design specifications and details to filter and trap sediment before it leaves the construction site.  (For more information, see SWPPP Guide, Chapter 4, ESC Principle 7.)

Also see, EPA’s Silt Fence BMP Fact Sheet at www.epa.gov/npdes/stormwater/menuofbmps/construction/silt_fences, or Fiber Rolls BMP Fact Sheet at www.epa.gov/npdes/stormwater/menuofbmps/construction/fiber_rolls

- Silt fencing will be installed at any downgrade location on the site where it is determined that flows from the site would impact an existing drainage, irrigation or neighboring property. Such fencing will be constructed according with weber county requirements and will include a collection swale on the upgrade side of the fencing for collection of sediments.

- A filtering method (straw bales, filtering fabric or an approved equal) will be in any drainage that may be impacted by the construction.  The filtering will be installed to intercept the flows away from the site and in the vicinity of the site where it is determined that the construction on the site would damage or degrade the quality of water flow.

- A clean up area will be established at the site for removal of mud or other debris that accumulates on the construction equipment prior to leaving the site.  The cleanup area shall be graded to prevent any contamination to adjacent properties, streets, and water ways by silt, mud, or other pollutants.

| BMP Description: Clean vehicles before leaving area |  |
| --- | --- |
| Installation Schedule: | Daily |
| Maintenance and Inspection: | Daily |
| Responsible Staff: | CJ Homes Inc. |

| BMP Description: Silt fences and barriers |  |
| --- | --- |
| Installation Schedule: | Beginning of project |
| Maintenance and Inspection: | Daily |
| Responsible Staff: | CJ Homes Inc. |

Repeat as needed

## 4.3	Retain Sediment On-Site

Instructions:

Describe sediment control practices (e.g., sediment trap or sediment basin), including design specifications and details (volume, dimensions, outlet structure) that will be implemented at the construction site to retain sediments on-site.  (For more information, see SWPPP Guide, Chapter 4, ESC Principle 8.)

Also, see EPA’s Sediment Basin BMP Fact Sheet at www.epa.gov/npdes/stormwater/menuofbmps/construction/sediment_basins

- Silt fencing will be installed at any downgrade location on the site where it is determined that flows from the site would impact an existing drainage, irrigation or neighboring property. Such fencing will be constructed according with weber county requirements and will include a collection swale on the upgrade side of the fencing for collection of sediments.  See SWPP Plan drawing

| BMP Description: Silt Fencing |  |
| --- | --- |
| Installation Schedule: | Beginning of project |
| Maintenance and Inspection: | Daily |
| Responsible Staff: | CJ Homes Inc. |

| BMP Description: Barriers |  |
| --- | --- |
| Installation Schedule: | Beginning of project |
| Maintenance and Inspection: | Daily |
| Responsible Staff: | CJ Homes Inc. |

Repeat as needed

## 4.4	Establish Stabilized Construction Exits

Instructions:

Describe location(s) of vehicle entrance(s) and exit(s), procedures to remove accumulated sediment off-site (e.g., vehicle tracking), and stabilization practices (e.g., stone pads or wash racks or both) to minimize off-site vehicle tracking of sediments and discharges to storm water. (For more information, see SWPPP Guide, Chapter 4, ESC Principle 9.)

Also, see EPA’s Construction Entrances BMP Fact Sheet at www.epa.gov/npdes/stormwater/menuofbmps/construction/cons_entrance

A temporary vehicle tracking control surface will be developed for vehicle ingress and egress.  The physical location will be located parallel and adjacent to 2200 South Street.  The tracking control surface dimensions will be 50 feet long and the width as required to support vehicle size.  The control surface will consist of 1.5” to 3” gravel over .5” to .75” filter layer.

| BMP Description: Clean vehicles before entering 2200 South Street/public road |  |
| --- | --- |
| Installation Schedule: | Before project begins |
| Maintenance and Inspection: | Daily |
| Responsible Staff: | CJ Homes Inc. |

| BMP Description: |  |
| --- | --- |
| Installation Schedule: |  |
| Maintenance and Inspection: |  |
| Responsible Staff: |  |

Repeat as needed

## 4.5	Protect Slopes

Instructions:

Describe controls (e.g., erosion control blankets, tackifiers) including design specifications and details that will be implemented to protect all slopes.  (For more information, see SWPPP Guide, Chapter 4, ESC Principle 5.)

Also, see EPA’s Geotextiles BMP Fact Sheet at www.epa.gov/npdes/stormwater/menuofbmps/construction/geotextiles

- Silt fencing will be installed at any downgrade location on the site where it is determined that flows from the site would impact an existing drainage, irrigation or neighboring property. Such fencing will be constructed according with weber county requirements and will include a collection swale on the upgrade side of the fencing for collection of sediments.  See SWPP Plan drawing

| BMP Description: Silt Fencing |  |
| --- | --- |
| Installation Schedule: | Prior to beginning of project |
| Maintenance and Inspection: | Daily |
| Responsible Staff: | CJ Homes Inc. |

| BMP Description: |  |
| --- | --- |
| Installation Schedule: |  |
| Maintenance and Inspection: |  |
| Responsible Staff: |  |

Repeat as needed

## 4.6	Stockpiled Sediment or Soil

Instructions:

Describe storm water controls and other measures you will take to minimize the discharge of sediment or soil particles from stockpiled sediment or soil.  Include a description of structural practices (e.g., diversions, berms, ditches, storage basins), including installation, and maintenance specifications, used to divert flows from stockpiled sediment or soil, retain or detain flows, or otherwise limit exposure and the discharge of pollutants from stockpiled sediment or soil.

Also, describe any controls or procedures used to minimize exposure resulting from adding to or removing materials from the pile.

- Silt fencing will be installed at any downgrade location on the site where it is determined that flows from the site would impact an existing drainage, irrigation or neighboring property. Such fencing will be constructed according with weber county requirements and will include a collection swale on the upgrade side of the fencing for collection of sediments.

- A filtering method (straw bales, filtering fabric or an approved equal) will be in any drainage that may be impacted by the construction.  The filtering will be installed to intercept the flows away from the site and in the vicinity of the site where it is determined that the construction on the site would damage or degrade the quality of water flow.

- A clean up area will be established at the site for removal of mud or other debris that accumulates on the construction equipment prior to leaving the site.  The cleanup area shall be graded to prevent any contamination to adjacent properties, streets, and water ways by silt, mud, or other pollutants.

| BMP Description: Silt fencing |  |
| --- | --- |
| Installation Schedule: | Prior to beginning project |
| Maintenance and Inspection: | daily |
| Responsible Staff: | CJ Homes Inc. |

| BMP Description: Filtering methods |  |
| --- | --- |
| Installation Schedule: | Prior to beginning project |
| Maintenance and Inspection: | daily |
| Responsible Staff: | CJ Homes Inc. |

Repeat as needed

## 4.7	Minimize Dust

Instructions:

Describe controls and procedures you will use at your project/site to minimize the generation of dust.

Minimal grading will occur to minimize dust.  Dry soil conditions will be maintained by controlling moisture content through hydration of soil/application of water

| BMP Description: Hydrate dry soil conditions |  |
| --- | --- |
| Installation Schedule: | During grading of roads |
| Maintenance and Inspection: | Daily/as required |
| Responsible Staff: | Rock Hard Construction |

| BMP Description: Hydrate dry soil conditions |  |
| --- | --- |
| Installation Schedule: | Excavating for piping |
| Maintenance and Inspection: | Daily/as required |
| Responsible Staff: | CJ Homes Inc. |

Repeat as needed

## 4.8	Topsoil

Instructions:

Describe how topsoil will be preserved and identify these areas and associated control measures on your site map(s).

If it is infeasible for you to preserve topsoil on your site, provide an explanation for why this is the case.

Topsoil will be preserved as much as possible.  During the completion of Phase 1, a common area/garden area will be developed with grass and shrubs.  See Landscaping plan.  Top soils will be used in this area as much as possible

| BMP Description: Utilize topsoil/reuse |  |
| --- | --- |
| Installation Schedule: | End of phase one |
| Maintenance and Inspection: | weekly |
| Responsible Staff: | CJ Homes Inc. |

| BMP Description: |  |
| --- | --- |
| Installation Schedule: |  |
| Maintenance and Inspection: |  |
| Responsible Staff: |  |

Repeat as needed

## 4.9	Soil Compaction

Instructions:

In areas where final vegetative stabilization will occur or where infiltration practices will be installed, describe the controls, including design, installation, and maintenance specifications that will be used to restrict vehicle or equipment access or condition the soil for seeding or planting.

Completion of Phase One a Common area will be developed which will include scrubs, tree, grass and a common garden area.  Landscaping drawing describes location and plan layout

| BMP Description: Development of Landscaping |  |
| --- | --- |
| Installation Schedule: | End of phase one |
| Maintenance and Inspection: | Weekly |
| Responsible Staff: | CJ Homes Inc. |

| BMP Description: |  |
| --- | --- |
| Installation Schedule: |  |
| Maintenance and Inspection: |  |
| Responsible Staff: |  |

Repeat as needed

## 4.10	High Altitude/Heavy Snows

Instructions:

See Part 2.1.2.i of the UCGP.  You must attempt to prepare for heavy snows by deploying storm water controls prior to the first heavy snow, and have appropriate storm water control measures designed to handle snow melt before heavy snows occur.

Stabilization measures should be deployed at the same time (See 2.2.1.c of the UCGP).

Completion of phase one will be in the middle of the summer of 2015.  Storm water control measures will be in place at the time through completion of detection pond located in the southwest corner of the project.  However, it is unlikely snow controls will be needed due to the time of year.

| Date Snow is Expected | Date of High Altitude/Heavy Snow Conditions BMPs to be Installed | Date of First Heavy Snow |
| --- | --- | --- |
| November 2015 | Scheduled: Jan 2016 |  |
|  | Actual: |  |

| BMP Description: Detention pone/barriers |  |
| --- | --- |
| Installation Schedule: | Phase 1 completion/July 2015 |
| Maintenance and Inspection: | Weekly |
| Responsible Staff: | CJ Homes Inc. |

| BMP Description: |  |
| --- | --- |
| Installation Schedule: |  |
| Maintenance and Inspection: |  |
| Responsible Staff: |  |

Repeat as needed

## 4.11	Linear Activities

Instructions:

See Part 2.1.2.b.i of the UCGP.  For linear projects, where you have determined that the use of perimeter controls in portions of the site is impracticable due to rights-of-ways, document why you believe this to be the case.

Description of why perimeter controls are not practicable.

- Perimeter controls are not practicable due to the work will be performed within the boundaries of the project.

## 4.12	Chemical Treatment

Instructions (see UCGP Parts 2.1.3.c and 7.2.9.b):

- If you are using treatment chemicals at your site, provide details for each of the items below.  This information is required as part of the SWPPP requirements in UCGP Part 7.2.9.b.

No treatment chemicals will be used

Soil Types

List all the soil types (including soil types expected to be found in fill material) that are expected to be exposed during construction and that will be discharged to locations where chemicals will be applied:  INSERT TEXT HERE

Treatment Chemicals

List all treatment chemicals that will be used at the site and explain why these chemicals are suited to the soil characteristics: INSERT TEXT HERE

Describe the dosage of all treatment chemicals you will use at the site or the methodology you will use to determine dosage: INSERT TEXT HERE

Provide information from any applicable Material Safety Data Sheets (MSDS): INSERT TEXT HERE

Describe how each of the chemicals will stored:  INSERT TEXT HERE

Include references to applicable state or local requirements affecting the use of treatment chemicals, and copies of applicable manufacturer’s specifications regarding the use of your specific treatment chemicals and/or chemical treatment systems: INSERT TEXT HERE

Special Controls for Cationic Treatment Chemicals (if applicable)

If you have been authorized by the Division of Water Quality to use cationic treatment chemicals, include the official DWQ authorization letter or other communication, and identify the specific controls and implementation procedures you are required to implement to ensure that your use of cationic treatment chemicals will not lead to a violation of water quality standards: INSERT (1) ANY LETTERS OR OTHER DOCUMENTS SENT FROM THE DWQ OFFICE CONCERNING YOUR USE OF CATIONIC TREATMENT CHEMICALS, AND (2) DESCRIPTION OF ANY SPECIFIC CONTROLS YOU ARE REQUIRED TO IMPLEMENT

Schematic Drawings of Storm Water Controls/Chemical Treatment Systems

Provide schematic drawings of any chemically-enhanced storm water controls or chemical treatment systems to be used for application of treatment chemicals: INSERT TEXT HERE

Training

Describe the training that personnel who handle and apply chemicals have received prior to permit coverage, or will receive prior to the use of treatment chemicals: INSERT TEXT HERE

## 4.13	Stabilize Soils

Instructions:

Describe controls (e.g., interim seeding with native vegetation, hydroseeding) to stabilize exposed soils where construction activities have temporarily or permanently ceased.  Also describe measures to control dust generation. Avoid using impervious surfaces for stabilization whenever possible.  (For more information, see SWPPP Guide, Chapter 4, ESC Principle 4.)

Also, see EPA’s Seeding BMP Fact Sheet at www.epa.gov/npdes/stormwater/menuofbmps/construction/seeding

At the completion of phase 1, a common area will be developed to stabilize soils which will include shrubs, grass, and vegetation.  Additionally, scrubs will be planted around the perimeter of the project to stabilize and beautify area.  All native vegetation will be undisturbed outside the road and lot areas.  See Landscaping map.  All measures to control dust will be through hydration of dry soils as required.

| BMP Description: Landscaping |  |
| --- | --- |
| x Permanent Temporary |  |
| Installation Schedule: | Completion of phase 1 |
| Maintenance and Inspection: | Weekly |
| Responsible Staff: | CJ Homes |

| BMP Description: Dust control |  |
| --- | --- |
| Permanent x Temporary |  |
| Installation Schedule: | Beginning of project |
| Maintenance and Inspection: | Daily |
| Responsible Staff: | CJ Homes Inc. |

Repeat as needed

## 4.14	Final Stabilization

Instructions:

Describe procedures for final stabilization.  If you complete major construction activities on part of your site, you can document your final stabilization efforts for that portion of the site (specific vegetative and/or non-vegetative practices). The UCGP allows you to then discontinue inspection activities in these areas.

You can amend or add to this section as areas of your project are finally stabilized.

Update your site plans to indicate areas that have achieved final stabilization.

Note that dates for areas that have achieved final stabilization should be included in Section 5, Part 5.1 of this SWPPP.

For more on this topic, see SWPPP Guide, Chapter 9.

At the completion of phase 1, a common area will be developed to stabilize soils which will include shrubs, grass, and vegetation.  Additionally, scrubs will be planted around the perimeter of the project to stabilize and beautify area.  All native vegetation will be undisturbed outside the road and lot areas.  See Landscaping map.  All measures to control dust will be through hydration of dry soils as required.

| BMP Description: Landscaping |  |
| --- | --- |
| Installation Schedule: | Completion of phase one |
| Maintenance and Inspection: | weekly |
| Responsible Staff: | CJ Homes Inc. |

| BMP Description: |  |
| --- | --- |
| Installation Schedule: |  |
| Maintenance and Inspection: |  |
| Responsible Staff: |  |

Repeat as needed

# SECTION 5: POLLUTION PREVENTION

Instructions:

Describe the key good housekeeping and pollution prevention (P2) BMPs that will be implemented to control pollutants in storm water (UCGP Part 2.3).

For more information, see SWPPP Guide, Chapter 5.

Consult your state’s or local jurisdiction’s design manual or resources in Appendix D of the SWPPP Guide.

For more information or ideas on BMPs, see EPA’s National Menu of BMPs http://www.epa.gov/npdes/stormwater/menuofbmps

Spills could be controlled by spreading plastic tarp under the pouring or mixing area.  Plastic tarp

will be gathered and disposed in the dumpster.

## 5.1	Spill Prevention and Response

Instructions:

Describe the spill prevention and control plan to include ways to reduce the chance of spills, stop the source of spills, contain and clean up spills, dispose of materials contaminated by spills, and train personnel responsible for spill prevention and control. (For more information, see SWPPP Guide, Chapter 5, P2 Principle 6.)

Some projects/site may be required to develop a Spill Prevention Control and Countermeasure (SPCC) plan under a separate regulatory program (40 CFR 112).  If you are required to develop an SPCC plan, or you already have one, you should include references to the relevant requirements from your plan.

Also, see EPA’s Spill Prevention and Control Plan BMP Fact sheet at www.epa.gov/npdes/stormwater/menuofbmps/construction/spill_control

Spills could be controlled by spreading plastic tarp under the pouring or mixing area.  Plastic tarp

will be gathered and disposed in the dumpster.

INSERT TEXT HERE or REFERENCE ATTACHMENT

Any discharges in 24 hours equal to or in excess of the reportable quantities listed in 40 CFR 117, 40 CFR 110, and 40 CFR 302 will be reported to the National Response Center and the Division of Water Quality (DWQ) as soon as practical after knowledge of the spill is known to the permittees.  The permittee shall submit within 14 calendar days of knowledge of the release a written description of: the release (including the type and estimate of the amount of material released), the date that such release occurred, the circumstances leading to the release, and measures taken and/or planned to be taken to the Division of Water Quality (DWQ), 288 North 1460 West, P.O. Box 144870, Salt Lake City, Utah 84114-4870.  The Storm Water Pollution Prevention Plan must be modified within14 calendar days of knowledge of the release to provide a description of the release, the circumstances leading to the release, and the date of the release.  In addition, the plan must be reviewed to identify measures to prevent the reoccurrence of such releases and to respond to such releases, and the plan must be modified where appropriate.

| Agency | Phone Number |
| --- | --- |
| National Response Center | (800) 424-8802 |
| Division of Water Quality ( DWQ) 24-Hr Reporting | (801) 538-6146 (801) 536-4123 |
| Utah Department of Health Emergency Response | (801) 580-6681 |

| Material | Media Released To | Reportable Quantity |
| --- | --- | --- |
| Engine oil, fuel, hydraulic & brake fluid | Land | 25 gallons |
| Paints, solvents, thinners | Land | 100 lbs (13 gallons) |
| Engine oil, fuel, hydraulic & brake fluid | Water | Visible Sheen |
| Antifreeze, battery acid, gasoline, engine degreasers | Air, Land, Water | 100 lbs (13 gallons) |
| Refrigerant | Air | 1 lb |

## 5.2	Construction and Domestic Waste

Instructions:

Describe measures (e.g., trash disposal, sanitary wastes, recycling, and proper material handling) to prevent the discharge of solid materials to receiving waters, except as authorized by a permit issued under section 404 of the CWA (For more information, see SWPPP Guide, Chapter 5, P2 Principle 1.)

Also, see EPA’s General Construction Site Waste Management BMP Fact Sheet at www.epa.gov/npdes/stormwater/menuofbmps/construction/cons_wasteman

- Solid waste disposal receptacles will be maintained on the site for disposal of materials.  A Porta Potty will be on site to service workers.

| BMP Description: Deposit waste in dumpsters |  |
| --- | --- |
| Installation Schedule: | During project |
| Maintenance and Inspection: | Daily |
| Responsible Staff: | CJ Homes Inc. |
| BMP Description: Human Waste/Porta Potty |  |
| Installation Schedule: | Beginning of project |
| Maintenance and Inspection: | Weekly |
| Responsible Staff: | CJ Homes Inc. |

## Repeat as needed

## 5.3	Washing of Applicators and Containers used for Concrete, Paint or Other Materials

Instructions:

Describe location(s) and controls to eliminate the potential for discharges from washout areas for concrete mixers, concrete washout, paint, stucco, mortar, drywall mud, and so on. (For more information, see SWPPP Guide, Chapter 5, P2 Principle 3.)

Also, see EPA’s Concrete Washout BMP Fact Sheet at www.epa.gov/npdes/stormwater/menuofbmps/construction/concrete_wash

Concrete and mortar products shall not be washed out into the street or drainage areas.  A designated containment facility or sufficient capacity to retain liquid and solid waste will be provided on the site.  Slurry from concrete or asphalt saw cutting shall be vacuumed or contained, dried, picked up and disposed of properly.  Additionally, a designated gravel/concrete wash out area of 4” rock with dimensions of 20’ by 40’ will be established to control discharge.

| BMP Description: Concrete/mortar products cleaning |  |
| --- | --- |
| Installation Schedule: | During project |
| Maintenance and Inspection: | Daily |
| Responsible Staff: | CJ Homes Inc. |

| BMP Description: |  |
| --- | --- |
| Installation Schedule: |  |
| Maintenance and Inspection: |  |
| Responsible Staff: |  |

Repeat as needed

## 5.4	Establish Proper Building Material Staging Areas

Instructions:

Describe construction materials expected to be stored on-site and procedures for storage of materials to minimize exposure of the materials to storm water. (For more information, see SWPPP Guide, Chapter 5, P2 Principle 2.)

Storage of materials will be located in designated area per SWPP plan map.  All storage areas will bermed for the containment of the materials

| BMP Description: Containing bulk materials |  |
| --- | --- |
| Installation Schedule: | Beginning of project |
| Maintenance and Inspection: | daily |
| Responsible Staff: | CJ Homes Inc. |

| BMP Description: |  |
| --- | --- |
| Installation Schedule: |  |
| Maintenance and Inspection: |  |
| Responsible Staff: |  |

Repeat as needed

## 5.5	Establish Proper Equipment/Vehicle Fueling and Maintenance Practices

Instructions:

Describe equipment/vehicle fueling and maintenance practices that will be implemented to control pollutants to storm water (e.g., secondary containment, drip pans, and spill kits).  UCGP Part 2.3.3.a

For more information, see SWPPP Guide, Chapter 5, P2 Principle 4.

Also, see EPA’s Vehicle Maintenance and Washing Areas BMP Fact Sheet at www.epa.gov/npdes/stormwater/menuofbmps/construction/vehicile_maintain

Spills could be controlled by spreading plastic tarp under the pouring or mixing area.  Plastic tarp

can be gathered and disposed in the dumpster

| BMP Description: Fueling of vehicles |  |
| --- | --- |
| Installation Schedule: | During project |
| Maintenance and Inspection: | Daily |
| Responsible Staff: | CJ Homes Inc. |

| BMP Description: |  |
| --- | --- |
| Installation Schedule: |  |
| Maintenance and Inspection: |  |
| Responsible Staff: |  |

Repeat as needed

## 5.6	Control Equipment/Vehicle Washing

Instructions:

Describe equipment/vehicle washing practices that will be used to minimize the discharge of pollutants from equipment and vehicle washing, wheel wash water, and other types of washing (e.g., locating activities away from surface waters and storm water inlets or conveyances and directing wash waters to a sediment basin or sediment trap, using filtration devices, such as filter bags or sand filters, or using other similarly effective controls).  (For more information, see SWPPP Guide, Chapter 5, P2 Principle 5.)

Describe how you will prevent the discharge of soaps, detergents, or solvents by providing either (1) cover (examples: plastic sheeting or temporary roofs) to prevent these detergents from coming into contact with rainwater, or (2) a similarly effective means designed to prevent the discharge of pollutants from these areas.

Also, see EPA’s Vehicle Maintenance and Washing Areas BMP Fact Sheet at www.epa.gov/npdes/stormwater/menuofbmps/construction/vehicile_maintain

A temporary vehicle tracking control surface will be developed for vehicle ingress and egress.  The physical location will be located parallel and adjacent to 2200 South Street.  The tracking control surface dimensions will be 50 feet long and the width as required to support vehicle size.  The control surface will consist of 1.5” to 3” gravel over .5” to .75” filter layer.

| BMP Description: Vehicle cleaning |  |
| --- | --- |
| Installation Schedule: | During project |
| Maintenance and Inspection: | daily |
| Responsible Staff: | CJ homes Inc. |

| BMP Description: |  |
| --- | --- |
| Installation Schedule: |  |
| Maintenance and Inspection: |  |
| Responsible Staff: |  |

Repeat as needed

## 5.7	Pesticides, Herbicides, Insecticides, Fertilizers, and Landscape Materials

Instructions:

Describe how you will comply with the UCGP Part 2.3.5 requirement to “minimize discharges of fertilizers containing nitrogen or phosphorus”.

No pesticides/herbicides will be used.  All landscape materials will be utilized in the common area.  All non-usable containers etc. will be disposed of in the dumpster.

| BMP Description: Landscape materials |  |
| --- | --- |
| Installation Schedule: | Completion of phase one |
| Maintenance and Inspection: | daily |
| Responsible Staff: | CJ Homes Inc. |

| BMP Description: |  |
| --- | --- |
| Installation Schedule: |  |
| Maintenance and Inspection: |  |
| Responsible Staff: |  |

Repeat as needed

## 5.8	Other Pollution Prevention Practices

Instructions:

Describe any additional BMPs that do not fit into the above categories.  Indicate the problem they are intended to address.

| BMP Description: |  |
| --- | --- |
| Installation Schedule: |  |
| Maintenance and Inspection: |  |
| Responsible Staff: |  |

| BMP Description: |  |
| --- | --- |
| Installation Schedule: |  |
| Maintenance and Inspection: |  |
| Responsible Staff: |  |

Repeat as needed

# SECTION 6: INSPECTIONS & CORRECTIVE ACTIONS

## 6.1	Inspections

Instructions:

Identify the individual(s) responsible for conducting inspections and ensure they are a “qualified person” per the UCGP Part 4.

The “qualified person” must meet the requirements of the UCGP, such as but not limited to the following:

- Utah Registered Storm Water Inspector (RSI)

- Certified Professional in Erosion and Sediment Control (CPESC)

- Certified Professional in Storm Water Quality (CPSWQ)

- Certified Erosion, Sediment, and Storm Water Inspector (CESSWI)

- Certified Inspector of Sediment and Erosion Control (CISEC)

- National Institute for Certification in Engineering Technologies, Erosion and Sediment Control, Level 3 (NICET)

- Utah Department of Transportation Erosion Control Supervisor (ECS)

Reference or attach the inspection form that will be used.

Describe the frequency that inspections will occur at your site including any correlations to storm frequency and intensity.

Increase in inspection frequency for sites discharging to Sensitive Waters (UCGP 4.1.3).

Note that inspection details for particular BMPs should be included in Sections 2 and 3.

You should also document the repairs and maintenance that you undertake as a result of your inspections.  These actions can be documented in the corrective action log described in Part 5.3 below.

For more on this topic, see SWPPP Guide, Chapters 6 and 8.

Also, see suggested inspection form in Appendix B of the SWPPP Guide.

1. Inspection Personnel: Identify the person(s) who will be responsible for conducting inspections and describe their qualifications:

Craig Standing General contractor/author of SWPP Plan

2. Inspection Schedule and Procedures:

Describe the inspection schedules and procedures you have developed for your site (include frequency of inspections for each BMP or group of BMPs, indicate when you will inspect, e.g., before/during/and after rain events, spot inspections):

Daily/weekly as previously described in the BMP per section

Describe the general procedures for correcting problems when they are identified. Include responsible staff and time frames for making corrections: Correct problems and bring back to standards prescribed in the SWPP Plan Craig Standing General contractor/author of SWPP Plan

Attach a copy of the inspection report you will use for your site.

REFERENCE ATTACHMENT

## Reduction in Inspection Frequency (if applicable)

For the reduction in inspections resulting from stabilization:  SPECIFY (1) LOCATIONS WHERE STABILIZATION STEPS HAVE BEEN COMPLETED AND (2) DATE THAT THEY WERE COMPLETED

For the reduction in inspections in arid, semi-arid, or drought-stricken areas:  INSERT BEGINNING AND ENDING DATES OF THE SEASONALLY-DEFINED ARID PERIOD FOR YOUR AREA OR THE VALID PERIOD OF DROUGHT

For reduction in inspections due to frozen conditions:  INSERT BEGINNING AND ENDING DATES OF FROZEN CONDITIONS ON YOUR SITE

## 6.2	Corrective Actions

Instructions:

Create here, or as an attachment, a corrective action log.  This log should describe repair, replacement, and maintenance of BMPs undertaken as a result of the inspections and maintenance procedures described above.  Actions related to the findings of inspections should reference the specific inspection report.

This log should describe actions taken, date completed, and note the person that completed the work.

Corrective Action Log:

INSERT LOG HERE or REFERENCE ATTACHMENT

## 6.3	Delegation of Authority

Instructions:

Identify the individual(s) or specifically describe the position where the construction site operator has delegated authority for the purposes of signing inspection reports, certifications, or other information.

Each inspection report must be signed in accordance with Appendix G, Part G.16 of the permit.

If a delegation letter is necessary, see Appendix K of this template and submit it to the Department and include in the SWPPP in Appendix K.

For more on this topic, see SWPPP Guide, Chapter 7.

Duly Authorized Representative(s) or Position(s):

Insert Name: Craig Standing

Insert Position: President

Insert Address: 5337 Ridgedale Drive

Insert City, State, Zip Code: Ogden UT 84403

Insert Telephone Number: 801 698 4048

Insert Fax/Email:

Attach a copy of the signed delegation of authority form in Appendix K.

# SECTION 7: TRAINING AND RECORDKEEPING

## 7.1	Training

Instructions:

Training your staff and subcontractors is an effective BMP.  As with the other steps you take to prevent storm water problems at your site, document that the personnel required to be trained in UCGP Part 6 completed the appropriate training.

The following personnel, at a minimum, must receive training, and therefore should be listed out individually in the table below:

  - Personnel who are responsible for the design, installation, maintenance, and/or repair of storm water controls (including pollution prevention measures);

  - Personnel responsible for the application and storage of treatment chemicals (if applicable);

  - Personnel who are responsible for conducting inspections as required in Part 4.1.1; and

  - Personnel who are responsible for taking corrective actions as required in Part 5.

Include dates, number of attendees, subjects covered, and length of training.

For more on this subject, see SWPPP Guide, Chapter 8.

Individual(s) Responsible for Training:

Craig Standing

Describe Training Conducted:

- General storm water and BMP awareness training for staff and subcontractors:

Rock Hard Construction/Ray Holmes

- Detailed training for staff and subcontractors with specific storm water responsibilities:

Rock Hard Construction/Ray Holmes

| Training Attendee Name | Title of Training | Duration | Date of Training |
| --- | --- | --- | --- |
| Ray Holmes | SWPP Plan | 1 hour | Before project begins |

Additional training documentation should be included in Appendix J.

## 7.2	Recordkeeping

Instructions:

The following is a list of records you should keep at your project site available for inspectors to review:

Dates of grading, construction activity, and stabilization (which is covered in Sections 2 and 3)

A copy of the construction general permit (attach)

The signed and certified NOI form or permit application form (attach)

A copy of the letter from EPA or/the state notifying you of their receipt of your complete NOI/application (attach)

Inspection reports (attach)

Records relating to endangered species and historic preservation (attach)

Check your permit for additional details

For more on this subject, see SWPPP Guide, Chapter 6.C.

Records will be retained for a minimum period of at least 3 years after the permit is terminated.

Date(s) when major grading activities occur:

INSERT LOG HERE or REFERENCE ATTACHMENT

Date(s) when construction activities temporarily or permanently cease on a portion of the site:

INSERT LOG HERE or REFERENCE ATTACHMENT

Date(s) when an area is either temporarily or permanently stabilized:

INSERT LOG HERE or REFERENCE ATTACHMENT

## 7.3	Log of Changes to the SWPPP

Instructions:

Create a log here, or as an attachment, of changes and updates to the SWPPP.  You should include additions of new BMPs, replacement of failed BMPs, significant changes in the activities or their timing on the project, changes in personnel, changes in inspection and maintenance procedures, updates to site maps, and so on.

Log of changes and updates to the SWPPP

INSERT LOG HERE or REFERENCE ATTACHMENT

# SECTION 8: WATER QUALITY

Instructions:

See Section 3 in UCGP.  Discharge must be controlled as necessary to meet applicable water quality standards.

If at any time you, or DWQ/MS4 inspector determined that your discharge is not being controlled as necessary to meet applicable water quality standard, you must take corrective actions as required in Part 5.2.1.  The corrective actions must be documented in this SWPPP as required in Part 5.2.2 and 5.4 (may compose an amendment and note in Amendment Log in 6.2 or Appendix G in this SWPPP).

Additional regulations may be imposed by the DWQ

## 8.1	UIC Class 5 Injection Wells

Instructions:

If you are using any of the following storm water controls at your site, as they are described below, you must document any contact you have had with DWQ for implementing the requirements for underground injection wells in the Safe Drinking Water Act and DEQ’s implementing regulation at UAC R317-7.

There may be additional local requirements related to such structures

Such controls (below) would generally be considered Class V UIC wells and all UIC Class V wells must be reported to DWQ for an inventory:

  - French Drains (if storm water is directed);

  - Commercially manufactured pre-cast or pre-built proprietary subsurface detention vaults, chambers, or other devices designed to capture and infiltrate storm water flow.

  - Drywells, seepage pits, or improved sinkholes (if storm water is directed).

For the State UIC Contact at DWQ call (801) 536-4300.

French Drain

Commercially Manufactured pre-cast or pre-built subsurface infiltration system

Drywell(s), seepage pit(s), improved sinkhole(s)

Description of your Class V Injection Well:

INSTERT DESCRIPTION AND/OR INCLUDE SPECIFICATIONS IN APPENDIX G

DWQ contact information:

Name:

Date:

Additional information:

Local Requirements:

## 8.2	Discharge Information

Instructions:

For Table 1, list the name of the first surface water(s) that would receive discharges from your site.  If your site has discharges to multiple surface waters, describe each as clearly as possible, such as Big Cottonwood Creek, a tributary to the Jordan River, and so on.

For Table 2, if any of the surface waters you listed out in Table 1 are listed as, provide specified information about pollutants causing the impairment and whether or not a Total Maximum Daily Load (TMDL) has been completed for the surface water that is applicable to construction sites.  For more information on TMDLs and impaired waters, including a list of TMDL contacts and links by state, visit http://www.waterquality.utah.gov/TMDL/ or www.epa.gov/npdes/stormwater/tmdl.  Your SWPPP should specifically include measures to prevent the discharge of these pollutants.

Your project will be considered to discharge to a Category 1 or 2 water if the first surface water to which you discharge is identified by the state as a Category 1 or 2 water.  For discharges that enter a storm sewer system prior to discharge, the first surface water to which you discharge is the water body that receives the storm water discharge from the storm sewer system.  Refer to Appendix C.

For more information, see SWPPP Guide, Chapter 3.A and 3.B.

Indicate the location of all waters, including wetlands, on the site map.

Note any stream crossings, if applicable.

List the storm sewer system or drainage system that storm water from your site could discharge to and the waterbody(s) that it ultimately discharges to.

Does your project/site discharge storm water into a Municipal Separate Storm Sewer System (MS4)? x Yes No

- List the MS4 that receives the discharge from the construction project: Taylor city

Are there any surface waters that are located within 50 feet of your construction disturbances? Yes x No

List the water body: INSERT TEXT HERE

## 8.3	Receiving Waters

- Table 1 – Names of Receiving Waters

Name(s) of the first surface water that receives storm water directly from your site and/or from the MS4. (note: multiple rows provided where your site has more than one point of discharge that flows to different surface waters)

- Taylor city

2.

3.

4.

5.

6.

## 8.4	Impaired Waters

|  | Is this surface water listed as "impaired"? | If you answered yes, then answer the following: |  |  |
| --- | --- | --- | --- | --- |
|  |  | What pollutant(s) are causing the impairment? | Has a TMDL been completed? | Pollutant(s) for which there is a TMDL |
| 1. | Yes x No |  | Yes No |  |
| 2. | Yes No |  | Yes No |  |
| 3. | Yes No |  | Yes No |  |
| 4. | Yes No |  | Yes No |  |
| 5. | Yes No |  | Yes No |  |
| 6. | Yes No |  | Yes No |  |

## 8.5	High Water Quality

- Table 3 – High Water Quality (Answer the following for each surface water listed in Table 1 above)

|  | Is this surface water designated as High Water Quality? (see Appendix C) | If you answered yes, specify which category the surface water is designated as? |
| --- | --- | --- |
| 1. | Yes x No | Category 1 Category 2 |
| 2. | Yes No | Category 1 Category 2 |
| 3. | Yes No | Category 1 Category 2 |
| 4. | Yes No | Category 1 Category 2 |
| 5. | Yes No | Category 1 Category 2 |
| 6. | Yes No | Category 1 Category 2 |

## 8.6	Dewatering Practices

Instructions:

If you will be discharging storm water that is removed from excavations, trenches, foundations, vaults, or other similar points of accumulation, include design specifications and details of all dewatering practices that are installed and maintained to comply with the UCGP Part 1.3.5.a and 2.1.3.d.

Construction dewatering is covered under UPDES permit UTG070000.  This applies to construction dewatering of uncontaminated storm water, groundwater, or surface water sources used in construction activities.  The permit, NOI and other associated information can be found at http://www.waterquality.utah.gov/UPDES/updes_f.htm.

| BMP Description: |  |
| --- | --- |
| Installation Schedule: |  |
| Maintenance and Inspection: |  |
| Responsible Staff: |  |

| BMP Description: |  |
| --- | --- |
| Installation Schedule: |  |
| Maintenance and Inspection: |  |
| Responsible Staff: |  |

Repeat as needed

## 8.7	Control Storm Water Flowing onto and through the Project

Instructions:

Describe structural practices (e.g., diversions, berms, ditches, storage basins) including design specifications and details used to divert flows from exposed soils, retain or detain flows, or otherwise limit runoff and the discharge of pollutants from exposed areas of the site.  (For more information, see SWPPP Guide, Chapter 4, ESC Principle 3.)

- Silt fencing will be installed at any downgrade location on the site where it is determined that flows from the site would impact an existing drainage, irrigation or neighboring property. Such fencing will be constructed according with weber county requirements and will include a collection swale on the upgrade side of the fencing for collection of sediments.

- A filtering method (straw bales, filtering fabric or an approved equal) will be in any drainage that may be impacted by the construction.  The filtering will be installed to intercept the flows away from the site and in the vicinity of the site where it is determined that the construction on the site would damage or degrade the quality of water flow.

- A clean up area will be established at the site for removal of mud or other debris that accumulates on the construction equipment prior to leaving the site.  The cleanup area shall be graded to prevent any contamination to adjacent properties, streets, and water ways by silt, mud, or other pollutants.

| BMP Description: Silt fencing |  |
| --- | --- |
| Installation Schedule: | Beginning of project |
| Maintenance and Inspection: | daily |
| Responsible Staff: | CJ Homes Inc. |

| BMP Description: filtering of water |  |
| --- | --- |
| Installation Schedule: | Beginning of project |
| Maintenance and Inspection: | daily |
| Responsible Staff: | CJ Homes Inc. |

Repeat as needed

## 8.8	Protect Storm Drain Inlets

Instructions:

Describe controls (e.g., inserts, rock-filled bags, or block and gravel) including design specifications and details that will be implemented to protect all inlets receiving storm water from the project during the entire project.  (For more information, see SWPPP Guide, Chapter 4, ESC Principle 6.)

Also, see EPA’s Storm Drain Inlet Protection BMP Fact Sheet at www.epa.gov/npdes/stormwater/menuofbmps/construction/storm_drain

- Silt fencing will be installed at any downgrade location on the site where it is determined that flows from the site would impact an existing drainage, irrigation or neighboring property. Such fencing will be constructed according with weber county requirements and will include a collection swale on the upgrade side of the fencing for collection of sediments.

- A filtering method (straw bales, filtering fabric or an approved equal) will be in any drainage that may be impacted by the construction.  The filtering will be installed to intercept the flows away from the site and in the vicinity of the site where it is determined that the construction on the site would damage or degrade the quality of water flow.

- A clean up area will be established at the site for removal of mud or other debris that accumulates on the construction equipment prior to leaving the site.  The cleanup area shall be graded to prevent any contamination to adjacent properties, streets, and water ways by silt, mud, or other pollutants.

| BMP Description: Silt fencing |  |
| --- | --- |
| Installation Schedule: | Beginning of project |
| Maintenance and Inspection: | daily |
| Responsible Staff: | CJ Homes Inc. |

| BMP Description: filtering water |  |
| --- | --- |
| Installation Schedule: | Beginning of project |
| Maintenance and Inspection: | daily |
| Responsible Staff: | CJ Homes Inc. |

Repeat as needed

# SECTION 9: POST-CONSTRUCTION BMPs

Instructions:

Describe all post-construction storm water management measures that will be installed during the construction process to control pollutants in storm water discharges after construction operations have been completed.  Examples of post-construction BMPs include the following:

- Biofilters

- Detention/retention devices

- Earth dikes, drainage swales, and lined ditches

- Infiltration basins

- Porous pavement

- Other proprietary permanent structural BMPs

- Outlet protection/velocity dissipation devices

- Slope protection

- Vegetated strips and/or swales

Identify any applicable federal, state, local, or tribal requirements for design or installation.

Describe how low-impact designs or smart growth considerations have been incorporated into the design.

For any structural BMPs, you should have design specifications and details and refer to them.  Attach them as appendices to the SWPPP or within the text of the SWPPP.

For more information on this topic, see your state’s storm water manual.

You might also want to consult one of the references listed in Appendix D of the SWPPP Guide.

Visit the post-construction section of EPA’s Menu of BMPs at: www.epa.gov/npes/menuofbmps

- Silt fencing will be installed at any downgrade location on the site where it is determined that flows from the site would impact an existing drainage, irrigation or neighboring property. Such fencing will be constructed according with weber county requirements and will include a collection swale on the upgrade side of the fencing for collection of sediments.

- A filtering method (straw bales, filtering fabric or an approved equal) will be in any drainage that may be impacted by the construction.  The filtering will be installed to intercept the flows away from the site and in the vicinity of the site where it is determined that the construction on the site would damage or degrade the quality of water flow.

- A clean up area will be established at the site for removal of mud or other debris that accumulates on the construction equipment prior to leaving the site.  The cleanup area shall be graded to prevent any contamination to adjacent properties, streets, and water ways by silt, mud, or other pollutants.

| BMP Description: Silt fencing |  |
| --- | --- |
| Installation Schedule: | Beginning of project |
| Maintenance and Inspection: | daily |
| Responsible Staff: | CJ Homes Inc. |

| BMP Description: filtering |  |
| --- | --- |
| Installation Schedule: | Beginning of project |
| Maintenance and Inspection: | daily |
| Responsible Staff: | CJ Homes Inc. |

Repeat as needed

# SECTION 10: CERTIFICATION

Instructions:

The SWPPP should be signed and certified by the construction operator(s).  Attach a copy of the NOI and a copy of the General Storm Water Permit for Construction Activity.  You can get a copy of the General Storm Water Permit for Construction Activity on the same web page that this template was obtained (www.waterquality.utah.gov/UPDES/stormwatercon.htm)

Professional/SWPPP Author

I certify under penalty of law that this document and all attachments were prepared under my direction or supervision in accordance with a system designed to assure that qualified personnel properly gathered and evaluated the information submitted.  Based on my inquiry of the person or persons who manage the system, or those persons directly responsible for gathering the information, the information submitted is, to the best of my knowledge and belief, true, accurate, and complete.  I am aware that there are significant penalties for submitting false information, including the possibility of fine and imprisonment for knowing violations.

| Name: | Craig Standing |  | Title: | President/ CJ Homes Inc. |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Signature: |  |  |  |  | Date: |  |

Repeat as needed for construction operator(s) at the site

# SWPPP APPENDICES

Attach the following documentation to the SWPPP:

## Appendix A – General Location Map

## Appendix B – Site Maps

## Appendix C – Construction General Permit

## Appendix D – NOI and Acknowledgement Letter from EPA/State/MS4

## Appendix E – Inspection Reports

## Appendix F – Corrective Action Log (or in Part 5.3)

## Appendix G – SWPPP Amendment Log (or in Part 7.4)

## Appendix H – Subcontractor Certifications/Agreements

## Appendix I – Grading and Stabilization Activities Log

## Appendix J – Training Log

## Appendix K – Delegation of Authority

## Appendix L – Additional Information (i.e. Documentation; other permits such as dewatering, stream alteration, wetland; and out of date swppp documents)

## Appendix M – BMP Specifications

Appendix F – Sample Corrective Action Log

Project Name:

SWPPP Contact:

| Inspection Date | Inspector Name(s) | Description of BMP Deficiency | Corrective Action Needed (including planned date/responsible person) | Date Action Taken/Responsible person |
| --- | --- | --- | --- | --- |

Appendix G – Sample SWPPP Amendment Log

Project Name:

SWPPP Contact:

| Amendment No. | Description of the Amendment | Date of Amendment | Amendment Prepared by [Name(s) and Title] |
| --- | --- | --- | --- |

Appendix H – Sample Subcontractor Certifications/Agreements

SUBCONTRACTOR CERTIFICATION

STORMWATER POLLUTION PREVENTION PLAN

Project Number:

Project Title:

Operator(s):

As a subcontractor, you are required to comply with the Storm water Pollution Prevention Plan (SWPPP) for any work that you perform on-site.  Any person or group who violates any condition of the SWPPP may be subject to substantial penalties or loss of contract.  You are encouraged to advise each of your employees working on this project of the requirements of the SWPPP.  A copy of the SWPPP is available for your review at the office trailer.

Each subcontractor engaged in activities at the construction site that could impact storm water must be identified and sign the following certification statement:

I certify under the penalty of law that I have read and understand the terms and conditions of the SWPPP for the above designated project and agree to follow the BMPs and practices described in the SWPPP.

This certification is hereby signed in reference to the above named project:

Company:

Address:

Telephone Number:

Type of construction service to be provided:

Signature:

Title:

Date:

Appendix I – Sample Grading and Stabilization Activities Log

Project Name:

SWPPP Contact:

| Date Grading Activity Initiated | Description of Grading Activity | Date Grading Activity Ceased (Indicate Temporary or Permanent) | Date When Stabilization Measures are Initiated | Description of Stabilization Measure and Location |
| --- | --- | --- | --- | --- |

Appendix J – Sample SWPPP Training Log

Storm water Pollution Prevention Training Log

Project Name:

Project Location:

Instructor’s Name(s):

Instructor’s Title(s):

Course Location: 	  Date:

Course Length (hours):

Storm water Training Topic:  (check as appropriate)

|  | Erosion Control BMPs |  | Emergency Procedures |
| --- | --- | --- | --- |
|  | Sediment Control BMPs |  | Good Housekeeping BMPs |
|  | Non-Storm water BMPs |  |  |

Specific Training Objective:

Attendee Roster:  (attach additional pages as necessary)

| No. | Name of Attendee | Company |
| --- | --- | --- |
| 1 |  |  |
| 2 |  |  |
| 3 |  |  |
| 4 |  |  |
| 5 |  |  |
| 6 |  |  |
| 7 |  |  |
| 8 |  |  |
| 9 |  |  |
| 10 |  |  |

Appendix K – Sample Delegation of Authority Form

Delegation of Authority

I, _______________________ (name), hereby designate the person or specifically described position below to be a duly authorized representative for the purpose of overseeing compliance with environmental requirements, including the Construction General Permit, at the ____________________________________ construction site.  The designee is authorized to sign any reports, storm water pollution prevention plans and all other documents required by the permit.

________________________________________ (name of person or position)

________________________________________ (company)

________________________________________ (address)

________________________________________ (city, state, zip)

________________________________________ (phone)

By signing this authorization, I confirm that I meet the requirements to make such a designation as set forth in ____________________________________ (Reference State Permit), and that the designee above meets the definition of a “duly authorized representative” as set forth in ____________________________________ (Reference State Permit).

I certify under penalty of law that this document and all attachments were prepared under my direction or supervision in accordance with a system designed to assure that qualified personnel properly gathered and evaluated the information submitted.  Based on my inquiry of the person or persons who manage the system, or those persons directly responsible for gathering the information, the information submitted is, to the best of my knowledge and belief, true, accurate, and complete.  I am aware that there are significant penalties for submitting false information, including the possibility of fine and imprisonment for knowing violations.

Name:

Company:

Title:

Signature:

Date: